Bluenica
‘Partner’ Temperature Sensor
Data Extraction

v.1.0
Apr 15, 2015
Author: M.W.

Copyright © bluenica Inc. 2013, 2015

Strictly Confidential

Table of Contents

Advertisement-based data collection	3

Advertised data structure	4

Short name AD section	4

Manufacturer-specific data AD section	5

Temperature record structure	6

Temperature value	6

Configuration record structure	7

Features	7

Configuration	7

Encryption	8

Sample advertisement processing code	9

# Advertisement-based data collection

HBTemp sensors periodically report their state using the BLE advertising technique.  The devices use the standard channels 37, 38, and 39 and broadcast the state with interval of 1s for the configured duration of broadcasting (typically 30s, 90s, or always.)  Bluenica sensors use the maximal allowed advertising packet (scan record) size – 31 bytes.

The advertising packet contains:

- bluenica device class information,

- device identification,

- actual data.

Device class information is stored in a “Local Short Name” AD section. Device identification and the actual data are stored in a “Manufacturer-specific data” AD section. Device identification (IEEE address) appears as clear text.  Other data is encrypted using a proprietary method.

Historically bluenica sensors have been broadcasting only the temperature information. WaSPs used to decode the temperature information, pack it, and submit to the servers in a very compact format.  Newer sensors are capable of broadcasting different types of data and WaSPs do not try to interpret the data anymore but store and upload it in verbatim form to the collecting servers.

# Advertised data structure

The bluenica temperature advertising packet (scan record) consists of two standard scan record AD sections: “Short name” and “Manufacturer-specific data”.  In many BLE Central implementations both AD sections are available as one 31-byte long byte buffer.

## Short name AD section

Section code: 0x08
Length: min. 3

| Offset | 0 | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- | --- |
| Item | header |  | Name |  |  |
| Sample | 4 | 08 | B | # | a |

| Item | Meaning |
| --- | --- |
| header | Short name AD header; length 4, type 0x08 |
| Name | Bluenica-specific 3 byte long device name, usually “B#a”. |

Bluenica sensors can be easily identified by checking if the second character of the advertised short device name is ‘#’.

The first character of the name defines the broadcasting device class:

- ‘A’ – a Controlling device, used to configure WaSPs and give them commands; data coming from ‘A’ devices is not collected but interpreted and executed on WaSPs;

- ‘B’, ‘b’ – Temperature Sensors; data coming from sensors is collected and uploaded;

- ‘C’, ‘c’ – Repeaters, used to extend the distance advertised information can be passed through; data coming from repeaters is collected and uploaded.

- ‘D’, ‘d’ – Data loggers; data coming from sensors is collected and uploaded;

Lower-case first character signifies ‘priority’ devices.  It is up to the application to apply meaning to ‘priority’.  Typically it means priority devices are processed first.

The third character of the name is defined at the moment to be ‘a’ and has no special meaning yet.

## Manufacturer-specific data AD section

A necessary condition for advertised data to be accepted as bluenica data is the presence of a “Manufacturer-specific data” section, with the manufacturer ID of 0x0D00.

Section code: 0xFF
Length: 21 to 25

| 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| header |  | ID |  | Address |  |  |  |  |  | D | T | Data 1 |  |  | Rec ID |  |  | Data 2 |  |  |  |  |  |  |  |
| 25 | xFF | x0D | x00 | xBE | xEE | x00 | x00 | x00 | x00 | 3 | 1 | 23 | 2 | 94 | 0 | 0 | 1 | 0 | 0 | 1 | 5 | 0 | 0 | 0 | 0 |

Bytes starting from offset 17 are encrypted.

| Item | Meaning |
| --- | --- |
| header | Manufacturer-specific data AD header; length 21-25, type 0xFF |
| ID | Manufacturer ID: 0x0D, 0x00 (Texas Instruments) |
| Address | bluenica device ID – 6 byte long IEEE Bluetooth board address |
| D | Time in minutes since the device has been last [D]ownloaded, max. 240 |
| T | MSB: partner encryption flag; 7 LSBs: data [T]ype |
| Data1 | Packed sensor data |
| Rec ID | Unique record ID |
| Data2 | Packed sensor data |

A test if the data is coming from a Partner sensor:

bool isPartner = advData[16] > 127;

Data type extraction:

byte dataType = (byte)(advData[16] & 0x7F); // mask off the ‘partner sensor’ bit

Data types:

public const int ADVERT_DATA_TYPE_NONE          = 0; // error?

public const int ADVERT_DATA_TYPE_TEMP          = 1; // Recorded Temperature

public const int ADVERT_DATA_TYPE_TEMP_AND_LIVE = 2; // Live Temperature (not used)

public const int ADVERT_DATA_TYPE_CONFIG        = 3; // Sensor Configuration

# Temperature record structure

| offset | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Item | Temp |  | Batt | Rec ID |  |  | Clock |  |  | Age | Unused |  |  |  |
| sample | 23 | 2 | 94 | 1 | 0 | 0 | 3C | 81 | 1A | 20 | 0 | 0 | 0 | 0 |

| Item | Meaning |
| --- | --- |
| Temp | Temperature value (in Celsius): integer part, decimal part |
| Batt | Battery level (%) |
| Rec ID | Data record ID |
| Clock | Device clock value at the time the record has been created, in seconds |
| Age | Data age (time since this record has been created), updated every 10 seconds |
| Unused | 4 available bytes in the broadcast buffer |

## Temperature value

int intPart = (sbyte)bytes[10]; // signed byte!

int decPart = (sbyte)bytes[11]; // signed byte!

Float Temperature = intPart + decPart / 100F;

# Configuration record structure

Every 144 records (typically once a day) bluenica sensors broadcast the device configuration instead of the recorded temperature.  The device configuration record contains the current temperature as well and can be used as a regular temperature record.

| Offset | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Item | Temp |  | F | Rec ID |  |  | Configuration |  |  |  | Major | Minor | Unused |  |
| sample | 23 | 2 | 9 | 1 | 0 | 0 | 30 | 6 | 10 | 15 | 2 | 52 | 0 | 0 |

| Item | Meaning |
| --- | --- |
| Temp | Temperature value (in Celsius): integer part, decimal part |
| F | Device features (bit flags) |
| Rec ID | Data record ID |
| Configuration | Device configuration |
| Major | Major code version component |
| Minor | Minor code version component |
| Unused | 2 available bytes in the broadcast buffer |

## Features

public const UInt32 FEATURE_ALWAYS_VISIBLE    = 0x00000001;

public const UInt32 FEATURE_ACCELEROMETER     = 0x00000002;

public const UInt32 FEATURE_ACCEL_WAKEUP      = 0x00000004;

public const UInt32 FEATURE_PRIORITY_DEVICE   = 0x00000008;

public const UInt32 FEATURE_ADV_LIVE_TEMP     = 0x00000010;

public const UInt32 FEATURE_WATCHDOG_DISABLED = 0x00000020;

## Configuration

UInt16 AdvVisibilityTimeoutSecs = (UInt16)(bytes[23] * 10),

UInt16 TempCheckItvSecs = (UInt16)(bytes[24] * 10),

UInt16 TempPeriodicWriteItvSecs = (UInt16)(bytes[25] * 60),

float  TempWriteOnDelta = bytes[26] / 10F

# Encryption

All Partner sensor data bytes starting from the global offset 17 are encrypted using a custom flipping and XORing method.

The byte value found at the global index 23 (in most data types the least-significant byte of the device internal clock) is used as the encryption dynamic seed.  The use of the least-significant byte of the device clock as the encryption seed guarantees that subsequent data records broadcast by the same device will have an entirely different content.

/// <summary>

/// Decrypts the advertised manufacturer-specific data coming from partner sensors.

/// Expects the bytes to start from the manufacturer code (right after LEN and FF)

/// Decryption is performed in place.

/// </summary>

/// <param name="bytes">Array of bytes</param>

private void DecryptPartnerAdvData(byte[] bytes)

{

if (bytes == null)

return;

const int globalEncStart = 17; // 17 relative to the complete advertising packet (which starts with the device name)

// data parts offsets

const int ofsDataType =  9;

const int ofsEncStart = 10;

const int ofsEncSeed  = 16;

if (bytes.Length >= globalEncStart)

{

byte seed = bytes[ofsEncSeed]; // seed - the only not encrypted byte

// bytes in the 'bytes' buffer starting from offset 10 need to be decrypted

for (int ofs = 0; ofs < bytes.Length - ofsEncStart; ofs++)

{

byte b = bytes[ofsEncStart + ofs];

byte add = (byte)(ofs + 7);

byte mod = (byte)(add ^ seed);

b = (byte)(b + ofs - mod);

bytes[ofsEncStart + ofs] = b;

}

// restore the original value of the seed

bytes[ofsEncSeed] = seed;

}

}

# Sample advertisement processing code

The C# code below illustrates a simple interpretation of advertised data on an Android platform.

// parsed out parameters

public long Address { get; set; }

public int DataType { get; set; }

public UInt32 RecId { get; private set; }

public float Temperature { get; private set; }

public int Battery { get; private set; }

public UInt32 Clock { get; private set; }

public int DownloadAge { get; private set; }

public int DataAge { get; private set; }

public int VersionMajor { get; private set; }

public int VersionMinor { get; private set; }

public HBTempSettings HBTempSettings;

/// <summary>

/// Parses advertised sensor information.

/// The function expects the bytes to start from the manufacturer code

/// (right after LEN and FF, at global offset of 7)

/// </summary>

/// <param name="bytes">Advertised bytes (already decrypted)</param>

private void ParseAdvertisedSensorInformation(byte[] bytes)

{

Address = BinUtils.PayloadBytesRevToTagId(bytes, 2);

// download age, data type

DownloadAge = bytes[8];

DataType = bytes[9];

// temperature appears in all existing data types

int intPart = (sbyte)bytes[10];

int decPart = (sbyte)bytes[11]; // signed!

Temperature = intPart + decPart / 100F;

// record number

RecId = BinUtils.PayloadBytesToUInt24(bytes, 13);

// information specific to data types

if (DataType == BNTagDefines.ADVERT_DATA_TYPE_CONFIG)

{

HBTempSettings = new HBTempSettings();

HBTempSettings.EnabledFeatures = bytes[12];

HBTempSettings.AdvVisibilityTimeoutSecs = (ushort)(bytes[16] * 10);

HBTempSettings.TempCheckItvSecs = (ushort)(bytes[17] * 10);

HBTempSettings.TempPeriodicWriteItvSecs = (ushort)(bytes[18] * 60);

HBTempSettings.TempWriteOnDelta = bytes[19] / 10F;

if (bytes.Length >= 22)

{

VersionMajor = bytes[20];

VersionMinor = bytes[21];

}

}

else // temperature or live temperature

{

// battery

Battery = bytes[12];

// clock, data age

Clock = BinUtils.PayloadBytesToUInt24(bytes, 16);

DataAge = bytes[19] * 10;

}

}

Used utility functions:

// Order of bytes: BE E0 00 00 30 10

public static long PayloadBytesRevToTagId(byte[] bytes, int ofs)

{

long cff = 1;

long id = 0;

for (int i = 5; i >= 0; i--)

{

id += bytes[ofs + i] * cff;

cff *= 256;

}

return id;

}

public static UInt32 PayloadBytesToUInt24(byte[] bytes, int ofs)

{

return (UInt32)(bytes[ofs] + bytes[ofs + 1] * 256 + bytes[ofs + 2] * 256 * 256);

}

public static UInt32 PayloadBytesToUInt32(byte[] bytes, int ofs)

{

UInt32 cff = 1;

UInt32 val = 0;

for (int i = 0; i < 4; i++)

{

val += bytes[ofs + i] * cff;

cff *= 256;

}

return val;

}