Bluenica WaSP

v.1.0
Mar 19, 2015
Author: Mirek Wojtowicz

Copyright © bluenica Inc. 2013, 2015

Strictly Confidential

Table of Contents

Advertisement-based data collection	3

Advertised data	4

Short name AD section	4

Manufacturer-specific data AD section	5

Advertisement processing code	6

Data collecting servers (Gateways)	7

UTC date/time API	7

Data collection API	8

Upload payload structure	10

XMIT record	10

Common header	10

Advertisement-based data	11

Binary data section	11

CRC value	12

Gateway response	13

# Advertisement-based data collection

WaSP devices, used to collect bluenica sensors data and upload it to Gateways, work in an endless scan-upload cycle.  The scan phase lasts for up to 5 minutes. During the scan phase WaSPs perform a passive BLE scan for 5 seconds, process found information, and continue scanning for 5 minutes.  After 5 minutes WaSPs attempt to upload collected data.  If the upload succeeds WaSPs return to the regular 5 minute long scanning phase. If the upload fails WaSPs revert to a shorter, 30 seconds long scanning phase and try to upload data again.

Bluenica HBTemp sensors periodically report their state using the BLE advertising technique.  The devices use the standard channels 37, 38, and 39 and broadcast the state with the interval of 1s for the configured duration of broadcasting (typically 30s, 90s, or always.)  Bluenica sensors use the maximal allowed advertising packet (scan record) size – 31 bytes.

The advertising packet contains:

- bluenica device class information (text),

- device identification (text),

- actual data (binary packed).

Device class information is stored in a standard “Local Short Name” AD section. Device identification and the actual data are stored in a standard “Manufacturer-specific data” AD section. Device identification (IEEE address) appears as clear text.  Other data is binary packed.

WaSP devices do not know the type or meaning of advertised data.  WaSPs detect that the data comes from bluenica sensors, store it and upload in verbatim form to the bluenica Gateways.

# Advertised data

The bluenica advertising packet (scan record) consists of two standard scan record AD sections: “Short name” and “Manufacturer-specific data”.  In most BLE Central implementations both AD sections are available to a scanning device as one 31-byte long byte buffer.  Other platforms (for example iOS) do not give access to the advertised data buffer but make it partially available through their own custom structures.  This document describes the complete advertised data buffer case.

## Short name AD section

Section code: 0x08
Length: min. 3

| Offset | 0 | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- | --- |
| Item | header |  | Name |  |  |
| Value | 4 | 08 | B | # | a |

| Item | Meaning |
| --- | --- |
| header | Short name AD header; length 4, type 0x08 |
| Name | Bluenica-specific 3 byte long device name, usually “B#a”. |

Bluenica sensors can be easily identified by checking if the second character of the advertised short device name is ‘#’.

The first character of the name defines the broadcasting device class:

- ‘A’ – a Controlling device, used to configure WaSPs and give them commands; data coming from ‘A’ devices is not collected but interpreted and executed on WaSPs;

- ‘B’, ‘b’ – Sensors, lower-case ‘b’ signifies ‘priority’ sensors; data coming from sensors is collected and uploaded;

- ‘C’ – Repeaters, used to extend the distance advertised information can be passed through; data coming from repeaters is collected and uploaded.

The third character of the name is defined at the moment to be ‘a’ and has no special meaning yet.

## Manufacturer-specific data AD section

Section code: 0xFF
Length: 21 to 25

| 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| header |  | ID |  | Address |  |  |  |  |  | D | T | Data1 |  |  | Rec ID |  |  | Data2 |  |  |  |  |  |  |  |
| 25 | FF | 0D | 00 | BE | EE | 00 | 00 | 00 | 00 | 13 | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

A necessary condition for advertised data to be accepted as bluenica data is the presence of a “Manufacturer-specific data” section, with the manufacturer ID of 0x0D00.

| Item | Meaning |
| --- | --- |
| header | Manufacturer-specific data AD header; length 21-25, type 0xFF |
| ID | Manufactured ID: 0x00, 0x0D (Texas Instruments) |
| Address | bluenica device ID – 6 byte long IEEE Bluetooth board address |
| D | Download age (time since the device has been last downloaded) |
| T | Data type |
| Data1 | Packed sensor data |
| Rec ID | Unique record ID |
| Data2 | Packed sensor data |

Data types:

- Temperature

- Live temperature (no longer used)

- Device configuration

All bytes starting from offset 15 (Download age) are sent to Gateways for processing.

# Advertisement processing code

The actual BLE scanning code varies significantly between different platforms and BLE drivers.  The advertising data processing however is similar regardless of the platform.  The original bluenica WaSP code running on a Texas Instruments BLE chipset is presented below.

WaSPs do not extract any parts of the advertised information. WaSP collect unique sensors records (where unique is defined by having either different IEEE address or a different record ID) and upload records to the servers.

// Identifies bluenica sensors, extracts and stores the advertised information.

void processAdvertisementData(gapDeviceInfoEvent_t* deviceInfo)

{

uint8* pData = deviceInfo->pEvtData; // pointer to scan record buffer start

uint8 dataLen = deviceInfo->dataLen;

uint8 adLen;

uint8 adType;

uint8* pEnd;

uint8 devName[4] = { 0, 0, 0, 0 };

pEnd = pData + dataLen - 1;

// while end of adv data not reached

while (pData < pEnd)

{

adLen = *pData++; // get length of next AD item

if (adLen > 0)

{

adType = *pData;

// if ad type is the device name

if (adType == GAP_ADTYPE_LOCAL_NAME_SHORT || adType == GAP_ADTYPE_LOCAL_NAME_COMPLETE)

{

if (pData[2] == '#') // "B#a" for HB, "A#a" for controllers, or old "B#BEE000123456"

{

devName[0] = pData[1]; // B

devName[1] = pData[2]; // #

devName[2] = pData[3]; // a

if (devName[0] == 'A')

{

// parse A# commands like Wi-Fi config

parseControllerCommands(pData + 3); // skip "A#"

}

}

}

else if (adType == GAP_ADTYPE_MANUFACTURER_SPECIFIC)

{

if (pData[1] == 0x0D && pData[2] == 0x00) // our custom data

{

if (isUtcResolved()) // no UTC - no data collection

if (devName[0] != 'A') // ignore controller commands

addAdvertisedData(pData + 3, 17, devName); // +3 points to the BdAddr of a sensor

}

}

// go to the next item

pData += adLen;

}

}

}

# Data collecting servers (Gateways)

WaSPs periodically upload all collected data to bluenica Gateways.  At the moment (Mar 2015) there exist two Gateways WaSPs can send data to:

http://bn-gateway.com

http://novotrace.com

Both Gateways operate on standard port 80.  Gateways serve two purposes: they collect bluenica data and also act as current UTC date/time servers.

WaSPs randomly select one of the Gateways for sending data for the load balancing purposes. Additionally whenever WaSPs encounter any upload HTTP errors they switch to the other Gateway to avoid data delays.

Gateways’ online status can be easily checked by sending the following HTTP GET request:

http://bn-gateway.com/exec.ashx?cmd=PING

The expected response is

err=0;text=PONG

The above test can be performed even in a regular browser.

## UTC date/time API

HBTemp sensors do not have a concept of current date/time. The sensors simply broadcast the current state.  It is the WaSP’s task to attach current date/time to advertised data.  UTC date/time is universally used at all steps of data collection.  WaSPs can easily obtain the current UTC date/time using the Gateways’ time API.  Gateways’ exec.ashx script detects the UTC request by finding the ‘cmd=utc’ command in the HTTP GET request. It sends the current UTC date/time back in a format specified in the ‘par’ parameter.

Syntax:

{gateway URL}/exec.ashx?cmd=utc&par=[0|1|2]

Examples:

http://bn-gateway.com/exec.ashx?cmd=utc&par=2 
http://novotrace.com/exec.ashx?cmd=utc&par=2

| Parameter | Response format |
| --- | --- |
| 0 | Binary response. It is the format used in the actual WaSPs code as it is the easiest one to parse on an embedded device. The response contains the current UTC date/time represented as 4 byte Unix Epoch value. Example: 0x0C 0x61 0x08 0x55 The above value represents 1426612492 which is Tue, 17 Mar 2015 17:14:52 GMT |
| 1 | Text response. The response contains a JSON string with CurTime field containing the current UTC expressed as a Unix Epoch value. Example: {"CurTime":1426612163} |
| 2 (default) | Text response. The response contains a JSON string with CurTime field containing the current UTC time in IEEE date/time format. Example: {"CurTime":"2015-03-17T17:14:00.000Z"} |

## Data collection API

Gateways accept sensor data through an HTTP POST request sent to a BinPost.ashx script.

Syntax:

{gateway URL}/BinPost.ashx?[fname=MyData][&dest=5][&crc=4]&sid=sessionId

Request parameters:

| Parameter | Description |
| --- | --- |
| fname | Optional file name / upload description. The parameter is a great place for identifying the WaSP, and passing any extra debugging information. Example – a real WaSP “fname” content: WF250_g0_AAA000001263_fb_s55_v3.40.29 “WF250” identifies the WaSP’s modem type “g0” informs which Gateway was used for sending data “AAA000001263” is the WaSPs Bluetooth IEEE address “fb” identifies the content (“full broadcast”) “s55” is the attached data size in bytes “v3.40.29” is the WaSPs code version What should we put here? |
| dest | Optional destination, a bitwise combination of: 1 - database, 4 – disk file The disk file option is only meaningful when sending data to “novotrace” Gateway. Default: 1 |
| crc | Optional size of CRC code appended to the uploaded data. Default 0 – no CRC. Recommended: 4 |
| sid | Mandatory unique 4 character long session ID randomly generated on a WaSP. The session ID is sent back in response for the modem data channel validation purposes. |

The order of parameters in the HTTP POST request is meaningless.

Examples:

http://bn-gateway.com/BinPost.ashx?ssid=ABCD

http://bn-gateway.com/BinPost.ashx?fname=MyData&sid=CAUS

http://bn-gateway.com/BinPost.ashx?fname=MyData&dest=5&crc=4&sid=1234

All collected data is sent in a POST request payload (see Upload payload structure).

# Upload payload structure

The POST request’s payload data (request body) consists of three sections:

- Transmission header (XMIT record)

- Binary data

- CRC value

Note:
Unless stated otherwise integer numbers are stored in the Little-endian format (the least significant byte in stored in the lowest address.)

## XMIT record

The main purpose of the XMIT data is the identification of the type of the content of the POST request payload.  WaSPs can send not only sensor data but also other types of information like diagnostics data or Bluetooth discovery statistics.  New types of data can be defined.

### Common header

Regardless of the XMIT record type they all share the same bluenica common header.  The header is used on a range of bluenica devices and two of its fields (Record ID and the Timestamp) have no meaning for XMIT records. All other fields are important and their values are listed in particular XMIT record type descriptions in the following sections.

| Name | Type | Size | Description |
| --- | --- | --- | --- |
| Record Syntax | uint8 | 1 | Record syntax version. Currently 1. |
| Record ID | uint8[3] | 3 | Pseudo-unique record ID. B0 is LSB. What should we put here? |
| Record type | uint8 | 1 | General record classification. |
| Record subtype | uint8 | 1 | Record subtype. The subtype field allows to define different records within one record type. |
| Timestamp | uint8[3] | 3 | Device clock value, in seconds. B0 is LSB. What is this? Time is generally 4 bytes. |
| Payload length | uint8 | 1 | The length of the following payload. Can be 0. |

Record types used on WaSPs:

| Identifier | Value | Description |
| --- | --- | --- |
| RECTYPE_TRANSMISSION | 2 | Data transmission between two bluenica devices |

Record Subtypes used on WaSPs:

| Identifier | Value | Description |
| --- | --- | --- |
| RECSUBTYPE_XMIT_SIMPLE | 1 | Simple transmission (data download) |
| RECSUBTYPE_XMIT_WASP_STATS | 2 | WaSP statistics (since v.1.35.52) |
| RECSUBTYPE_XMIT_WASP_LOG | 3 | WaSP activity log (since v.1.44.63) |
| RECSUBTYPE_XMIT_ADV_TEMP_DATA | 4 | Advertisement-based Temperature Data (since v. 2.10.76) |
| RECSUBTYPE_XMIT_ADV_DATA | 5 | Advertisement-based data (since v. 3.30) |

### Advertisement-based data

The record is created on central devices and marks a block of records containing data collected from a number of sensors via the advertised information.

| Name | Type | Size | Description |
| --- | --- | --- | --- |
| Header | N/A | 10 | Common record header, with the following variable values: Record type = RECTYPE_TRANSMISSION Record subtype = RECSUBTYPE_XMIT_ADV_DATA Payload length = 16 |
| Gateway_ID | uint8[6] | 6 | The ID of the gateway device. What should we put here? |
| Gateway_ID | uint8[6] | 6 | The ID of the gateway device. What should we put here? How is it different from above? |
| Date/time | uint32 | 4 | UNIX-style epoch value of the current UTC date/time. Example: 1347980168 is Tue, 18 Sep 2012 14:56:08 GMT Endianess?? |

## Binary data section

The Advertisement-based data binary data section consists of three parts:

- header

- index of sensors

- data records

In order to conserve limited WaSP memory data records do not contain full addresses of sensors the records come from.  An index of all discovered sensors is built in memory and a one byte sensor ID is assigned to each sensor and used in data records.  Therefore an individual advertised data upload is limited to data coming from up to 256 sensors.  The code ignores data coming from additional sensors once the index is full.

#### Header

| Name | Type | Size | Description |
| --- | --- | --- | --- |
| # of tags | uint8 | 1 | Number of tags the data will come from |
| # of records | uint32 | 4 | Total number of data records Endianess? |
| Records size | uint8 | 1 | The size of an individual data record. In WaSP v.2.10.76 the size is 14. Future versions may append more data to the records but the first 14 bytes will remain fixed. |

#### Index of Tags

A number of records of the following structure:

| Name | Type | Size | Description |
| --- | --- | --- | --- |
| Tag_ID | uint8[6] | 6 | The ID of the peripheral (Tag) device. Is this bytes 9-14 in the 0xFF? |
| Tag Index | uint8 | 1 | An ordinal number assigned to the Tag in this set of data. |

#### Data records

A number of records of the following structure:

| Name | Type |  | Size | Description |  |
| --- | --- | --- | --- | --- | --- |
| Tag index | uint8 |  | 1 | The ID of the peripheral (Tag) device. |  |
| Date/time | uint32 | 4 |  | UNIX-style epoch value of the current UTC date/time. Example: 1347980168 is Tue, 18 Sep 2012 14:56:08 GMT Endianess? |
| Name | char[2] | 2 |  | The first and third character of the device name. Example: for “B#a” “Ba” is stored. |
| Data | int8[16] |  | 16 | Full advertised data starting from offset 15 (Download age) |  |

## CRC value

CRC value is a simple 4 byte sum of all bytes in the payload (XMIT and Data) and can be calculated as follows.

uint32 crc32 = 0;

for (int i = 0; i < payloadSize; i++)

{

crc32 += payload[i];

}

CRC value is appended to the payload in LSB-MSB byte order.

Note:
The “Content-Length” field of the HTTP POST header must account
for the CRC bytes in the request body.

# Gateway response

Gateways send back a binary response to all received data uploads.  A valid response starts with “BN” characters after which a number of pairs Key-Value may follow.  The order of Key-Value pairs is meaningless.  The first byte of the value specifies the remaining value length.

For convenience Gateway responses may contain the current UTC date/time in Unix Epoch format.  This date/time can be used for calibrating the WaSP’s internal clock.

WaSPs safely ignore keys they have no knowledge about.

The following code illustrates a sample processing of the Gateway responses.

#define GrKey_Nop     0

#define GrKey_Err     1

#define GrKey_Sid     2

#define GrKey_Crc     3

#define GrKey_Utc     4

#define HTTP_ERR_NOERROR      0

#define HTTP_ERR_DATA_SHORT   1

#define HTTP_ERR_CRC_ERROR    2

#define HTTP_ERR_DB_FAILURE   3

#define HTTP_ERR_SSID_ERROR   254 // no valid SID found in the response

#define HTTP_ERR_UNAVAILABLE  255 // no valid response received

// Parses the response, returns the success information.

// The response means success if the error code is 0 (HTTP_ERR_NOERROR)

// and the session ID matches the expoected sid.

// Response syntax:

// "BN" ErrKey ErrLen Err SidKey SidLen sid CrcKey CrcLen Crc32

uint8 parseResponse(char* resp, uint16 respLen, char* sid)

{

uint8 err = HTTP_ERR_UNAVAILABLE;

uint8 sidOk = 0;

if (respLen >= 11 && resp[0] == 'B' && resp[1] == 'N')

{

int pos = 2;

int key = 0;

while (pos < respLen && key >= 0)

{

key = resp[pos++];

uint8 len = resp[pos++];

switch (key)

{

case GrKey_Err:

err = resp[pos];

break;

case GrKey_Sid:

sidOk = 1;

for (uint8 j = 0; j < osal_strlen(sid); j++)

{

if (resp[pos + j] != sid[j])

{

sidOk = 0;

break;

}

}

break;

case GrKey_Utc:

{

// update UTC

uint32 utc;

osal_memcpy(&utc, resp + pos, 4);

setUtcTime(utc);

}

break;

}

pos += len;

}

}

uint8 success = (sidOk && err == HTTP_ERR_NOERROR);

return success;

}

Sample response without UTC:

resp[0] = 'B';

resp[1] = 'N';

resp[2] = 1; // err

resp[3] = 1;

resp[4] = 0;

resp[5] = 2; // sid

resp[6] = 4;

resp[7] = sid[0];

resp[8] = sid[1];

resp[9] = sid[2];

resp[10] = sid[3];

resp[11] = 3; // crc

resp[12] = 4;

resp[13] = 156;

resp[14] = 76;

resp[15] = 13;

resp[16] = 19;