HFPD (Hands Free Pager Device)

Daemon Specification

ITW Weigh Wrap

401 W Market St.

Troy, OH  45374

10/23/2018

# VERSION HISTORY

| Version # | Implemented By | Revision Date | Approved By | Approval Date | Reason |
| --- | --- | --- | --- | --- | --- |
| 1.0 | Randy Blankley | 10/23/2019 |  |  | Initial version |

Table of Contents

1	Overview	5

1.1	Requirements	5

2	Communication	5

3	Messages	6

3.1	PING (100)	6

3.1.1	Version 1	6

3.2	PONG (101)	6

3.2.1	Version 1	6

3.3	DEV_LIST_REQ (200)	7

3.3.1	Version 1	7

3.4	DEV_LIST (201)	7

3.4.1	Version 1	7

3.5	DEV_OPNAME_LIST (202)	8

3.5.1	Version 1	8

3.6	DEV_FLASH_FIRMWARE_OTA (220)	9

3.6.1	Version 1	9

3.7	DEV_FIRMWARE_REQUEST (221)	9

3.7.1	Version 1	9

3.8	DEV_FIRMWARE_PROGRESS (222)	10

3.8.1	Version 1	10

3.9	DEV_CONNECT (300)	11

3.9.1	Version 1	11

3.10	DEV_DISCONNECT (301)	11

3.10.1	Version 1	11

3.11	DEV_READ_ALL_CONFIG (302)	12

3.11.1	Version 1	12

3.12	DEV_READ_CONFIG (303)	12

3.12.1	Version 1	12

3.13	DEV_WRITE_CONFIG (304)	13

3.13.1	Version 1	13

3.14	DEV_CONFIG (305)	13

3.14.1	Version 1	13

3.15	DEV_SEND_MESSAGE (306)	14

3.15.1	Version 1	14

3.16	DEV_CLEAR_ALL_MESSAGES (307)	15

3.16.1	Version 1	15

3.17	DEV_CMD_RESULT (350)	15

3.17.1	Version 1	15

3.18	DEV_CMD_STATE (351)	16

3.18.1	Version 1	16

3.19	DEV_CMD_ERROR (352)	16

3.19.1	Version 1	16

4	Connection Process	17

5	Appendices	18

5.1	Result Codes	18

5.2	Configuration Id Values	18

# Overview

The HFPD (Hands Free Pager Device) Daemon software resides on a scale or wrapper as a service and monitors BLE (Bluetooth Low Energy) transmissions looking for HFPD devices.  HFPD devices are designed to extend the functionality of Hobart equipment by allowing users to perform hands free login, contact supervisors, and receive diagnostic messages.

The HFPD service is responsible for tracking device signal levels, modifying device configuration, and updating device firmware.  This document outlines technical details of communicating with this service to perform the above functionality.

## Requirements

The HFPD Daemon shall be an app installed onto scale and/or wrapper devices via a feature file.  For the scale/wrapper device software to interact with the service the scale/wrapper must be licensed via the SCALE_HANDSFREEDEVICE license.  This license shall be available through license key generator upon launch of the product.

The HFPD Daemon shall instantiate a TCP server on port 5100 and listen for active connections on the loopback adapter only (it is not open to devices outside of the scale or wrapper).

# Communication

The HFPD Daemon actively accepts multiple connections via TCP server.  Packets to/from the server have the format outlined below.  All values sent to/from the server shall be in network byte order, most significant byte is first.

Packet Structure:

| [4] | [3] | [1] | [Variable] |
| --- | --- | --- | --- |
| Message Length | Message Type | Message Version | Message Payload |

Message length represents how many bytes constitute this message.  Messages may or may not have a payload.  For a message with no payload message length is 4 (i.e. it only has type and version).  For a message with payload the payload size is Message length - 4.

To read an entire message from a TCP socket the following pseudo code may be followed:

IF at least 4 bytes are pending on the socket:

MessageLen := Read 4 bytes from socket

IF at least MessageLen bytes are pending on the socket:

Message := Read MessageLen bytes off the socket

Process Message

Each message type/version will have its own payload size and format specification.  Each message type is outlined below.

Any client connecting to the HFPD Daemon is required to answer PING requests.  Failure to answer PING requests in a timely manner will result in the HFPD Daemon disconnecting the client.

# Messages

Technical details of each message type are listed below.

## PING (100)

### Version 1

Message Type: 100

Message Version: 1

Payload Size: 0

This message is sent by the HFPD server or a client device and shall be immediately answered by a PONG reply message.

## PONG (101)

### Version 1

Message Type: 101

Message Version: 1

Payload Size: 0

This message is sent by the HFPD server or a client device and is a reply to the PING message.

## DEV_LIST_REQ (200)

### Version 1

Message Type: 200

Message Version: 1

Payload Size: 5

Payload Structure:

| [1] | [4] |
| --- | --- |
| TX Power | Age |

This message is sent by a client device to request a listing of current HFPD devices seen by the service.  The HFPD service will reply with a DEV_LIST message in response.

TX Power represents the advert strength you are looking for, where:

- 0 = HIGH_POWER_ADVERT, you are requesting devices which have recently sent a high power advert

- 1 = LOW_POWER_ADVERT, you are requesting devices which have recently sent a low power advert

Age represents how long ago the device had sent an advert, in seconds.  For example, if you sent 10 you will receive a list of devices which sent an advert in the last 10 seconds.

## DEV_LIST (201)

### Version 1

Message Type: 201

Message Version: 1

Payload Size: Variable

Payload Structure:

| [2] | [2] | [2] | [Variable] |
| --- | --- | --- | --- |
| Result | Strength | Count | Devices |

This message is sent from the HFPD server to a client in response to a DEV_LIST_REQ message. It contains a list of device addresses and names.

Result is a result code.  See appendix for all result code values.

Strength is the TX power that was requested.

Count is the number of devices found.  For each device found the following structure will be repeated:

| [Variable] | [1] | [1] | [2] | [1] |  | [Variable] |
| --- | --- | --- | --- | --- | --- | --- |
| Address | RSSI | Battery Level | Firmware Ver. | Num Messages | Oper. Name |  |

Address is a NULL terminated string with the device address.

RSSI is the device signal strength from -128 to 127.

Battery Level is how charged the battery is from 0 to 100.

Firmware Version is the device firmware version where the most significant byte is major version and the least significant byte is minor version.

Num Messages is how many unread messages are on the device.

Oper. Name is the NULL terminated operator name string.

## DEV_OPNAME_LIST (202)

### Version 1

Message Type: 202

Message Version: 1

Payload Size: Variable

Payload Structure:

| [2] | [2] | [Variable] |
| --- | --- | --- |
| Result | Count | Operators |

This message is sent from the client device to the HFPD server and contains operator names for all devices.  The HFPD Daemon will associate the operator names with each device.

Result is a result code.  See appendix for all result code values.

Count is the number of operators in the message.  For each operator the following structure will be repeated:

| [Variable] | [Variable] |
| --- | --- |
| Address | Oper. Name |  |

Address is a NULL terminated string with the device address.

Oper. Name is the NULL terminated operator name string.

## DEV_FLASH_FIRMWARE_OTA (220)

### Version 1

Message Type: 220

Message Version: 1

Payload Size: Variable

Payload Structure:

| [Variable] | [4] | [Variable] |
| --- | --- | --- |
| Address | OTA Id | Filename |

This message is sent from a client device to to the HFPD server to begin an OTA firmware upgrade.  The device must be in OTA mode for the process to start.  Once this message is sent the HFPD Daemon will look for the device in OTA mode for approximately 30 seconds.  If the device does not enter OTA mode within this period the service will give up and the message must be resent.

Address is the NULL terminated device address that is being upgraded.

OTA Id is the over-the-air identification code of the device.

Filename is the NULL terminated fully pathed out firmware file name that resides on the HFPD server.  This filename will be validated before anything is sent to the device.

## DEV_FIRMWARE_REQUEST (221)

### Version 1

Message Type: 221

Message Version: 1

Payload Size: Variable

Payload Structure:

[Variable]

Address

This message is sent from the HFPD server to each connected client when a HFPD device enters OTA upgrade mode.

Address is the NULL terminated device address that is in OTA mode.

## DEV_FIRMWARE_PROGRESS (222)

### Version 1

Message Type: 222

Message Version: 1

Payload Size: Variable

Payload Structure:

| [Variable] | [1] | [1] | [2] |
| --- | --- | --- | --- |
| Address | Start Flag | Finish Flag | Progress |

This message is sent from the HFPD server to the client that sent an DEV_FLASH_FIRMWARE_OTA message that contains the progress of the OTA upgrade.  As the upgrade proceeds over time this message will be sent as progress changes.

Address is the NULL terminated device address that is being upgraded.

The Start flag is 1 when the OTA upgrade is starting, otherwise its 0.

The Finish flag is 1 when the OTA upgrade is finished, otherwise its 0.  The upgrade may or may not have failed… to determine this check the progress.  If its 100 then the device is fully upgraded and the process completed successfully.

Progress is a value 0 to 100 that indicates how far along the OTA upgrade is.

## DEV_CONNECT (300)

### Version 1

Message Type: 300

Message Version: 1

Payload Size: Variable

Payload Structure:

[Variable]

Address

This message is sent from a client to the HFPD server that indicates it wishes to establish a BLE connection to a device.  See below for BLE connection process… the connect process is asynchronous and further commands may not work until the device is fully connected and ready.

Address is the NULL terminated device address.

## DEV_DISCONNECT (301)

### Version 1

Message Type: 301

Message Version: 1

Payload Size: Variable

Payload Structure:

[Variable]

Address

This message is sent from a client to the HFPD server that indicates it wishes to disconnect from a BLE device.  A connection must be established before this message may be sent.

Address is the NULL terminated device address.

## DEV_READ_ALL_CONFIG (302)

### Version 1

Message Type: 302

Message Version: 1

Payload Size: Variable

Payload Structure:

[Variable]

Address

This message is sent from a client to the HFPD server that indicates it wants all configuration values from an HFPD BLE device.  A connection must be established before this message may be sent.

Address is the NULL terminated device address.

## DEV_READ_CONFIG (303)

### Version 1

Message Type: 303

Message Version: 1

Payload Size: Variable

Payload Structure:

| [Variable] | [1] | [2] |
| --- | --- | --- |
| Address | Cache Flag | Config Id |

This message is sent from a client to the HFPD server that indicates it wants a single configuration value from an HFPD BLE device.  A connection must be established before this message may be sent.

Address is the NULL terminated device address.

Cache flag will attempt to read the value from a local in-memory copy when set to 1.  If the config value is not cached or when Cache Flag is 0 it will query the device directly through BLE.

Config Id represents the config value you wish to read.  See appendix for all config values.

## DEV_WRITE_CONFIG (304)

### Version 1

Message Type: 304

Message Version: 1

Payload Size: Variable

Payload Structure:

| [Variable] | [2] | [2] | [Variable] |
| --- | --- | --- | --- |
| Address | Config Id | Config Value Length | Config Value |

This message is sent from a client to the HFPD server that indicates it wants to write a configuration value to an HFPD BLE device.  A connection must be established before this message may be sent.

Address is the NULL terminated device address.

Config Id represents the config value you wish to write.  See appendix for all config values.

Config value length is how many bytes the configuration value is.

Config value is the actual value you wish to send.

## DEV_CONFIG (305)

### Version 1

Message Type: 305

Message Version: 1

Payload Size: Variable

Payload Structure:

| [Variable] | [1] | [2] | [2] | [Variable] |
| --- | --- | --- | --- | --- |
| Address | Cache Flag | Config Id | Config Value Length | Config Value |

This message is sent from the HFPD server to client and contains a configuration value.

Address is the NULL terminated device address.

Cache flag is 1 when the value was pulled from the in-memory configuration cache.  It will be 0 when the value came directly from the device.

Config Id represents the config value.  See appendix for all config values.

Config value length is how many bytes the configuration value is.

Config value is the actual value.

## DEV_SEND_MESSAGE (306)

### Version 1

Message Type: 306

Message Version: 1

Payload Size: Variable

Payload Structure:

| [Variable] | [1] | [1] | [1] | [1] | [Variable] |
| --- | --- | --- | --- | --- | --- |
| Address | Hour | Minute | Second | Sender | Text |

This message is sent from a client to the HFPD to send a text message to a device.  A connection must be established before this message may be sent.

Address is the NULL terminated device address.

Hour is the hour in which the text message was sent.

Minute is the minute in which the text message was sent.

Second is the second in which the text message was sent.

Sender is the numeric id of who sent the message, usually a scale or wrapper number.

Text is a NULL terminated string containing the text message you wish to send, up to 16 characters.

## DEV_CLEAR_ALL_MESSAGES (307)

### Version 1

Message Type: 307

Message Version: 1

Payload Size: Variable

Payload Structure:

[Variable]

Address

This message is sent from a client to the HFPD to clear all messages off a device.  A connection must be established before this message may be sent.

Address is the NULL terminated device address.

## DEV_CMD_RESULT (350)

### Version 1

Message Type: 350

Message Version: 1

Payload Size: Variable

Payload Structure:

| [3] | [2] | [Variable] |
| --- | --- | --- |
| Command | Result Code | Address |

This message is sent from a HFPD to a client device in response to a request message.  For example, when sending a HFPD_DEV_READ_CONFIG message for an unknown device you will receive this message as a response.

Command is the message type you sent to the HFPD service.

Result code is the command result.  See appending for all result codes.

Address is the NULL terminated device address, if applicable.

## DEV_CMD_STATE (351)

### Version 1

Message Type: 351

Message Version: 1

Payload Size: Variable

Payload Structure:

| [Variable] | [1] |
| --- | --- |
| Address | State |

This message is sent from the HFPD to a client device to indicate device state.

Address is the NULL terminated device address.

State is the device state.

- 0 = STATE_DISCONNECTED

- 1 = STATE_CONNECTING

- 2 = STATE_CONNECTED

- 3 = STATE_READY

## DEV_CMD_ERROR (352)

### Version 1

Message Type: 352

Message Version: 1

Payload Size: Variable

Payload Structure:

| [Variable] | [2] | [Variable] |
| --- | --- | --- |
| Address | Error Code | Error Description |

This message is sent from the HFPD to a client device that indicates a device error.

Address is the NULL terminated device address.

Error code is the numeric value of the error.

Error description is a textual description of the error.

# Connection Process

Note that when connecting to HFPD BLE devices this process is asynchronous.  Furthermore, the device must indicate its ready before any subsequent command may be issued.  The below chart shows connecting to a HFPD BLE device, requesting a config value, then disconnecting.

Client

HFPD Server

DEV_LIST_REQ, High power last 10 seconds

DEV_LIST

DEV_CONNECT

DEV_CMD_RESULT, SUCCESS

HFPD_DEV_STATE, CONNECTING

HFPD_DEV_STATE, CONNECTED

HFPD_DEV_STATE, READY

DEV_READ_CONFIG, Not Cached

DEV_CMD_RESULT, SUCCESS

DEV_CONFIG

DEV_DISCONNECT

DEV_CMD_RESULT, SUCCESS

HFPD_DEV_STATE, DISCONNECTED

# Appendices

## Result Codes

The following result codes are present in many response messages.

- 0 = ERROR_SUCCESS

No Error.

- 1 = ERROR_BAD_REQUEST

Bad Request.

- 2 = ERROR_UNKNOWN_DEVICE

Unknown Device.

- 3 = ERROR_NOT_CONNECTED

Not connected to device.

- 4 = ERROR_UNSUPPORTED

Unsupported command.

- 5 = ERROR_CONFIG_UNKNOWN

Unknown Config Value.

- 6 = ERROR_CONFIG_REQUESTED

Configuration was Requested.

- 7 = ERROR_INVALID_FIRMWARE

Firmware is invalid.

## Configuration Id Values

The following configuration values are supported on the HFPD device.

- 0 = FIRMWARE_VER [2 bytes]

Read Only, Firmware Version.

- 1 = DEVICE_TYPE [2 bytes]

Read Only, Device Type.

- 2 = OTA_ID [4 bytes]

Read Only, OTA Number (for firmware upgrade).

- 10 = MSG_ACTIVE_TOP [10 bytes]

Top Line Message when Active.

- 11 = MSG_ACTIVE_BOTTOM [10 bytes]

Bottom Line Message when Active.

- 12 = MSG_INACTIVE_TOP [10 bytes]

Top Line Message when Inactive (off shift).

- 13 = MSG_INACTIVE_BOTTOM [10 bytes]

Bottom Line Message when Inactive (off shift).

- 20 = USER_NAME [16 bytes]

Operator user name.

- 21 = SHIFT_DURATION [1 byte]

Operator shift duration in hours: [0-36], 0 to disable

- 50 = ADVERT_TX_POWER_LEVEL [1 byte]

Low power advert tx strength (-40, -20, -16, -12, -8, -4, 0, 4).

- 51 = ADVERT_TX_COUNT [1 byte]

Low power advert count before high power: [0-255], 0 to disable

- 52 = ADVERT_TX_DELAY [2 bytes]

Delay between adverts in milliseconds: [100-10000]

- 60 = BEEP_TIME [1 byte]

Beep time in milliseconds: [0-255], 0 to disable

- 80 = LOGO [768 bytes]

Write Only, Logo bitmap. The first bit is the bottom left corner of the bitmap. Bits are send from bottom to top, left to right.