# Vantron HFPD Messaging

# Bluetooth Low Energy (BLE) Advertisements

The HFPD broadcasts general device information using BLE adverts. These adverts contain a 25-byte payload separated into five information blocks. Each block is identified with an 8-bit eir identification code.

## Information Blocks

| [0 - 1] | [2-3] | [4-15] | [16-19] | [20-24] |
| --- | --- | --- | --- | --- |
| Flags | Tx Power Level | Manufacturer Specific Data | Service Data | Shortened Local Name |

## Flags

Total Size: 2 Bytes

Data Type ID: 0x01

| [0] | [1] |
| --- | --- |
| 0x01 | Flags |

Flags conform to BLE EIR advertising standards. See Bluetooth Core Specification:Vol. 3, Part C, section 8.1.3 (v2.1 + EDR, 3.0 + HS and 4.0)Vol. 3, Part C, sections 11.1.3 and 18.1 (v4.0)Core Specification Supplement, Part A, section 1.3 for more information on flags.

## Tx Power Level

Total Size: 2 Bytes

Data Type ID: 0x0A

| [2] | [3] |
| --- | --- |
| 0x0A | Tx Power Level |

HFPD sends n low power adverts followed by a high power advert. The Tx Power level byte indicates the power level at which the advert was sent. Possible power levels are -40, -20, -16, -12, -8, -4, 0, and 4 dBm

## Manufacturer Specific Data

Total Size:  12 Bytes

Data Type ID: 0xFF

| [4] | [5-6] | [7-12] | [13-14] | [15] |
| --- | --- | --- | --- | --- |
| 0xFF | Device Type | MAC Address | FW Version | Number of Unread Messages |

The manufacturer specific data block contains 4 sets of information. The device type field contains either 0xAFF1 or 0xAFF0. Devices of type 0xAFF1 are equipped with a EInk display while devices of type 0xAFF0 do not. Bytes 7-12 contain the Bluetooth MAC address of the device. The firmware version is contained in bytes 13 and 14. Byte 13 contains the major revision while byte 14 contains the minor revision. e.g. 0x0103 is firmware version 1.3. The last byte of the manufacturer specific data contains the number of unread messages on the device.

## Service Data

Total Size: 4 Bytes

Data Type ID: 0x16

| [16] | [17-18] | [19] |
| --- | --- | --- |
| 0x16 | Battery Level ID | Battery Level |

Service Data is used to broadcast the current battery level as a percent. The maximum value for the Battery level is 0x64 (100).

## Shortened Local Name

Total Size: 5 Bytes

Data Type ID: 0x08

| [20] | [21-24] |
| --- | --- |
| 0x08 | ITWH |

All devices are given local name of ITWH. This is to identify a device as a Hobart HFPD.

# Messaging

## Introduction

The HFPD device is equipped with two characteristics to allow for the transmission and reception of messages.

## Receive

The characteristic with the UUID of {0000fff8-0000-1000-8000-00805f9b34fb } is the Rx characteristic. The Rx characteristic accepts messages of 20 bytes, where the first byte is the command id and the last byte is a CRC checksum. The remaining bytes are used to transfer information to the device. The commands are outlined in the commands section.

## Transmit

The characteristic with the UUID of {0000fff7-0000-1000-8000-00805f9b34fb} is the Tx characteristic. It is used to notify the user of successful or failed commands. Each packet will be 20 bytes long. The first byte will be the command id. The last byte will contain a CRC checksum. In the event of a failed command, the Tx characteristic will respond with the command id followed by the byte 0x99

## Commands

All commands must be 20 bytes long.

### Get Firmware Version

Command ID: 0x01

Upon reception of this message the device will reply with a message containing the firmware version of the device.

Command:

| [0] | [1-18] | [19] |
| --- | --- | --- |
| 0x01 | N/A | CRC Checksum |

Reply:

| [0] | [1] | [2] | [3-18] | [19] |
| --- | --- | --- | --- | --- |
| 0x01 | FW Major Revision | FW Minor Revision | N/A | CRC Checksum |

### Get OTA Number

Command ID: 0x02

Upon reception of this message the device will reply with a message containing the OTA Number of the device.

Command:

| [0] | [1-18] | [19] |
| --- | --- | --- |
| 0x02 | N/A | CRC Checksum |

Reply:

| [0] | [1-4] | [5-18] | [19] |
| --- | --- | --- | --- |
| 0x01 | OTA Number | N/A | CRC Checksum |

### Go to OTA Mode

Command ID: 0x03

OTA Mode is not supported at this time.

### Receive Message

Command ID: 0x04

This message adds a new notification message to the devices message queue and alerts the user of the reception of a new message. Must send Set IP and Time before sending Receive Message.

Command:

| [0] | [1-17] | [18] | [19] |
| --- | --- | --- | --- |
| 0x04 | uint8_t * Message | pad | CRC Checksum |

### Set IP and Time

Command ID: 0x05

Sets the IP and Time for the next message sent. Must be sent prior to sending a Receive message. Time is displayed in the HH:MM:SS format.

Command:

| [0] | [1] |  | [2] |  | [3] | [4] | [5-18] | [19] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x05 | uint8_t IP | uint8_t Hours |  | uint8_t Min |  | uint8_t Sec | N/A | CRC Checksum |

### Clear Messages

Command ID: 0x06

Clears all messages from the devices message queue.

Command:

| [0] | [1-18] | [19] |
| --- | --- | --- |
| 0x06 | N/A | CRC Checksum |

### Set Active Message Top

Command ID: 0x07

Sets the top line of the device active message. This message is restricted to ten characters due to screen size constraints.

Command:

| [0] | [1-11] | [12-18] | [19] |
| --- | --- | --- | --- |
| 0x07 | uint8_t * Message | pad | CRC Checksum |

### Set Active Message Bottom

Command ID: 0x08

Sets the bottom line of the device active message. This message is restricted to ten characters due to screen size constraints.

Command:

| [0] | [1-11] | [12-18] | [19] |
| --- | --- | --- | --- |
| 0x08 | uint8_t * Message | pad | CRC Checksum |

### Set Sleeping Message Top

Command ID: 0x09

Sets the top line of the device inactive message. This message is restricted to ten characters due to screen size constraints.

Command:

| [0] | [1-11] | [12-18] | [19] |
| --- | --- | --- | --- |
| 0x09 | uint8_t * Message | pad | CRC Checksum |

### Set Active Sleeping Bottom

Command ID: 0x0A

Sets the bottom line of the device inactive message. This message is restricted to ten characters due to screen size constraints.

Command:

| [0] | [1-11] | [12-18] | [19] |
| --- | --- | --- | --- |
| 0x0A | uint8_t * Message | pad | CRC Checksum |

### Set User Name

Command ID: 0x0B

Sets The user name for the device to the given value. The user name is displayed in the message field when there are no unread messages and the device is active.

Command:

| [0] | [1-17] | [18] | [19] |
| --- | --- | --- | --- |
| 0x0B | uint8_t * User Name | pad | CRC Checksum |

### Set Tx Power Level

Command ID: 0x0C

Sets the TX power level for the device. Accepted values are: -40, -20, -16, -12, -8, -4, 0, 4

Command:

| [0] | [1] | [2-18] | [19] |
| --- | --- | --- | --- |
| 0x0C | int8_t Tx Level | N/A | CRC Checksum |  |

### Set Shift Duration

Command ID: 0x0D

Sets the number of hours before automatic shutdown of the device. The timer is started when the device is powered on. Setting Shift Duration to 0 disables the shutdown timer. The maximum value for Shift Duration is 36 hours.

| [0] | [1] | [2-18] | [19] |
| --- | --- | --- | --- |
| 0x0D | uint8_t shift duration | N/A | CRC Checksum |

### Set Low Power Counts

Command ID: 0x0E

Sets the number of low power adverts sent before a high power advert is sent. Setting low power counts to 0 disables high power adverts. The maximum value for low power counts is 255.

| [0] | [1] | [2-18] | [19] |
| --- | --- | --- | --- |
| 0x0E | uint8_t counts | N/A | CRC Checksum |

### Set Advert Delay

Command ID: 0x0F

Sets the delay in milliseconds between advert broadcasts. The minimum delay is 100ms. The maximum delay is 10000ms.

| [0] | [1] | [2-18] | [19] |
| --- | --- | --- | --- |
| 0x0F | uint8_t delay | N/A | CRC Checksum |

### Set Beep Time

Command ID: 0x10

Sets the duration in milliseconds of the beep tone. Setting beep time to 0 disables the beeper. The maximum value for beep time is 255ms.

| [0] | [1] | [2-18] | [19] |
| --- | --- | --- | --- |
| 0x10 | uint8_t duration | N/A | CRC Checksum |

### Set New Logo

Command IDs: 0x50-0x80

To set the logo a series of commands need to be sent. Sending 0x50 signals the start of the logo transfer and contains the first 16 bytes of the logo bitmap. The commands should then be sent in sequence until 0x80 is sent, signaling the end of the new logo. Once 0x80 is received the logo will be written to flash Each bit represents a pixel on the screen. Where bit 0 is the bottom left corner of the logo. The bits should be sent from bottom to top, left to right.

| [0] | [1-17] | [18] | [19] |
| --- | --- | --- | --- |
| 0x50 - 0x80 | uint8_t Logo[i - 0x50] | pad | CRC Checksum |

#### Logo Example: