# New Gen Wrapper System Messaging

Aaron Swift, Alex Zavala, Carlos Guzman

2/16/2015

New wrapping system CAN context diagram

#candump of bootsequence:

can0  000   [1]  FF			<-- open communication interface

can0  000   [2]  01 01                                 	--> communication interface open

can0  004   [5]  06 00 10 80 00      	<-- read vendor id

can0  004   [2]  12 8C                                 	--> hobart vendor id

can0  004   [5]  06 00 10 80 02 		<-- read product id

can0  004   [2]  04 01                                 	--> weigher product id

can0  004   [5]  06 00 13 FF E0                 	<-- read major software version

can0  004   [2]  01 00                                 	--> weigher major software version

can0  004   [5]  06 00 13 FF E2                 	<-- read minor software version

can0  004   [2]  00 00                                 	--> weigher minor software version

can0  006   [2]  00 00                                 	--> read board node id

can0  006   [1]  00                                      	<-- board node id

can0  006   [3]  01 00 04                 	--> program board node id

can0  006   [1]  00                                      	<-- programmed OK

can0  006   [2]  00 00                                 	--> read board node id

can0  006   [1]  04                                      	<-- board node id

can0  006   [4]  02 10 82 00                      	--> jump to weigher application

Exiting bootloader

can0 000 [1] FF 			<-- open communication interface

can0 000 [2] 01 01			 --> communication interface open

can0 004 [5] 06 00 10 80 00 		<-- read vendor id

can0 004 [2] 12 8C 			--> hobart vendor id

can0 004 [5] 06 00 10 80 02 		<-- read product id

can0 004 [2] 02 05 			--> printer product id

can0 004 [5] 06 00 13 FF E0 		<-- read major software version

can0 004 [2] 01 00			 --> printer major software version

can0 004 [5] 06 00 13 FF E2 		<-- read minor software version

can0 004 [2] 00 00 			--> printer minor software version

can0 006 [2] 00 00 			--> read board node id

can0 006 [1] 00 			<-- board node id

can0 006 [3] 01 00 01 			--> program board node id (first printer)

can0 006 [1] 00 			<-- programmed OK

can0 006 [2] 00 00 			<-- read board node id

can0 006 [1] 01 			--> board node id

can0 006 [4] 02 10 82 00 		--> jump to weigher application

Exiting bootloader

CanManager->probeBus()

#### CAN messaging schemes

The arbitration field is 11 bits wide which gives us a range of COB IDs from 0 – 7FF.

- Each Hobart peripheral message greater than 8 bytes is spread to the individual receive mailboxes.

Peripheral will round robin message boxes until complete message is transferred.

Each Hobart peripheral individual message will have a unique COBID (message box). Messages greater than 8 bytes will send the same ID until message transfer is complete. Scheme 2 was chosen due to ease of implementation.

- Each Hobart peripheral message will be start with a header COBID that will contain the type of message the size of the message and the starting and ending COBID’s. This is the same as scheme 1 but with overhead of a message header sent first. Message header format would be Message type, size and the start and stop COBID’s.

The SAM7x CAN interface has 8 mailboxes which can be programmed for TX/RX and a range of COB ID’s can be assigned to each RX mailbox.

1 mailbox for TX

7 mailboxes for RX

## Weigher:

#### To Weigher

Field Size (Bytes)

WGMode:

ultimaWgMode.function_code

ultimaWgMode.mode 								Total:	1

WgControl:

ultimaWgControl.function_code

ultimaWgControl.control								Total:	1

WgRequestStatus:								Total: 	1

WgRequestConfig: (Inquire)							Total:	1

WgRequestWakeup								Total: 	1

WgEnable									Total: 	1

WGDisable									Total: 	1

WgRezero									Total: 	1

WgReset									Total: 	1

WgColdReset									Total: 	1

WgConfig:

usbWgConfig.function_code

usbWgConfig.configType 								2

usbWgConfig.config.ultimaWgCfg.center_of_maintenance_zone				4	 usbWgConfig.config.ultimaWgCfg.scale_factor 						4

usbWgConfig.config.ultimaWgCfg.max_weight 						4

usbWgConfig.config.ultimaWgCfg.gain_factor 						4

usbWgConfig.config.ultimaWgCfg.zero_reference 					4

usbWgConfig.config.ultimaWgCfg.calibration_reference 					4

usbWgConfig.config.ultimaWgCfg.rate_option_time 					2

usbWgConfig.config.ultimaWgCfg.auto_zero_maintenance_limit 				2

usbWgConfig.config.ultimaWgCfg.prepack_motion_count				2

usbWgConfig.config.ultimaWgCfg.initialize_zero_time 					2

usbWgConfig.config.ultimaWgCfg.auto_zero_time 					2

usbWgConfig.config.ultimaWgCfg.rate_option_limit 					2

usbWgConfig.config.ultimaWgCfg.zero_reference_limit 					2

usbWgConfig.config.ultimaWgCfg.calibration_reference_limit 				2

usbWgConfig.config.ultimaWgCfg.small_motion_limit 					2

usbWgConfig.config.ultimaWgCfg.large_motion_limit 					2

usbWgConfig.config.ultimaWgCfg.large_motion_count					2 usbWgConfig.config.ultimaWgCfg.small_motion_count 					2

usbWgConfig.config.ultimaWgCfg.no_motion_count					2 usbWgConfig.config.ultimaWgCfg.zero_tracking_option 					2

usbWgConfig.config.ultimaWgCfg.negative_weight_option				2 usbWgConfig.config.ultimaWgCfg.powerup_maintenance_zone_limit 			2

usbWgConfig.config.ultimaWgCfg.operate_maintenance_zone_limit 			2

usbWgConfig.config.ultimaWgCfg.filter_speed 						1

usbWgConfig.config.ultimaWgCfg.reference_voltage					1

usbWgConfig.config.ultimaWgCfg.last_calibration_date  					4

usbWgConfig.config.ultimaWgCfg.number_of_calibrations 				2

usbWgConfig.config.ultimaWgCfg.last_configuration_date 				4

usbWgConfig.config.ultimaWgCfg.number_of_configurations 				2

usbWgConfig.config.weigher_type  							1

usbWgConfig.config.flags 								1

usbWgConfig.config.min_weight_to_print 						4

usbWgConfig.config.weigher_model  							1

Total:	83

** Note message function code no longer necessary the message is now defined by the CAN COB ID.

83 / 8 = 10.5 total transfers

#### From Weigher:

Message Size (Bytes)

WgWakeup:

UINT8       function_code;

UINT8       pad0;

UINT8       wgCfgStatus;								Total: 	1

_newWeigherWgStatus:

UINT8       function_code;

UINT8       pad0;

UINT16      status;									2

UINT32      counts;									4

UINT32      raw_counts;									4

UINT32      filtered_uncalibrated_counts;							4

UINT32      AD_counts;									4

Total:	18

_usbWgInquire:

UINT8           function_code;

UINT8           pad0;

UINT16         configType;									2

UINT8       device_signature[SystemDevSignatureSize];

UINT32      center_of_maintenance_zone;							4

UINT32      scale_factor;									4

UINT32      max_weight;             /* in calibrated counts */					4

UINT32      gain_factor;									4

UINT32      zero_reference;								4

UINT32      calibration_reference;								4

UINT16      rate_option_time;               /* in seconds */					2

UINT16      auto_zero_maintenance_limit;							2

UINT16      prepack_motion_count;						   	2

UINT16      initialize_zero_time;           /* in seconds */					2

UINT16      auto_zero_time;                 /* in seconds */					2

UINT16      rate_option_limit;								2

UINT16      zero_reference_limit;   							2

UINT16      calibration_reference_limit;   							2

UINT16      small_motion_limit;								2

UINT16      large_motion_limit;								2

UINT16      large_motion_count;								2

UINT16      small_motion_count;								2

UINT16      no_motion_count;								2

UINT16      zero_tracking_option;								2

UINT16      negative_weight_option;							2

UINT16      powerup_maintenance_zone_limit;						2

UINT16      operate_maintenance_zone_limit;						2

UINT8       filter_speed;									1

UINT8       reference_voltage;								1

UINT32      last_calibration_date;								4

UINT16      number_of_calibrations;							2

UINT32      last_configuration_date;							4

UINT16      number_of_configurations;							2

UINT8       ntep_cc[ntep_ccSize];								1

UINT32      calibration_gain_coefficient;							4

UINT32      calibration_offset_coefficient;							4

UINT8       weigher_type;								1

UINT8       load_cell_id;									1

UINT8       flags;										1

UINT32     min_weight_to_print;								4

UINT8       weigher_model;								1

Total: 	96

System

SysWakeup:

UINT8	function_code;

UINT8	pad0;

UINT8	sysCfgStatus;								Total:	1

SysStatus:	(Breakup message)

UINT8 function_code;

UINT8 pad0;

UINT16 bit_error;										2

UINT16 address_error;									2

UINT16 rom_checksum;									2

UINT32 flags;										4

UINT8 partno[PART_NO_SIZE];								1

UINT16 system_checksum;								2

UINT32 init_flags;									4

UINT16 software_major_revision;								2

UINT16 software_minor_revision;								2

UINT8 hardware_major_revision;  // Device ID						1

UINT8 hardware_minor_revision;              							1

UINT32 weigher_serial_num;   								4

UINT32 DebugFlags;									4

Total:	31

WgReqSerialNumber								Total:	1

WgSerialNumber									Total:	4

WgReqVersion									Total:	1

WgVersion									Total:	6

SysConfig: (Inquire)	No longer needed

UINT8	 function_code;

UINT8	pad0;

UINT16	configType;									2

UINT8	device_signature[SystemDevSignatureSize];

DeviceAddress	station_address;

DeviceAddress	controller_address;

#### CAN Messages:

WgRequestWakeup		COB ID: 400					Total: 	1

WgWakeup			COB ID: 401					Total:	1

WgRequestStatus 		COB ID: 402					Total:	1

WgStatus			COB ID: 403					Total: 	18

COB ID: 403

COB ID: 403

WgReqSerialNumber		COB ID: 404					Total:	1

WgSerialNumber 			COB ID: 405 					Total:	4

WgReqVersion			COB ID: 406					Total:	1

WgVersion			COB ID: 407					Total:	1

WgRequestConfig: (Inquire)	COB ID: 408					Total:	1

WgConfig (usbWgInquire)		COB ID: 409					Total: 	91

COB ID: 409

COB ID: 409

COB ID: 409

COB ID: 409

COB ID: 409

COB ID: 409

COB ID: 409

COB ID: 409

COB ID: 409

COB ID: 409

COB ID: 409

WgWriteConfig(WgConfig)	COB ID: 40A					Total: 	84

COB ID: 40A

COB ID: 40A

COB ID: 40A

COB ID: 40A

COB ID: 40A

COB ID: 40A

COB ID: 40A

COB ID: 40A

COB ID: 40A

COB ID: 40A

WgMode			COB ID: 40B

WgEnable			COB ID: 40C					Total: 	1

WgDisable			COB ID: 40D					Total: 	1

WgRezero			COB ID: 40E					Total: 	1

WgReset			COB ID: 40F					Total: 	1

WgColdReset			COB ID: 410					Total: 	1

WgControl			COB ID: 411					Total: 	1

#### Can Message Format:			MSB -> LSB

## Printer

#### From Printer:								Message Size (Bytes)

** Primary Printer is assigned 		100 – 1D8 for COB ID’s

Secondary Printer is assigned 		200 – 2D8

Third Printer is assigned 		300 – 3D8

PrWakeup:

UINT8	prEvent;

UINT8	pad0;

UINT8	printerType;									1

UINT8	labelWidth;									1

UINT16	bufferSize;									2

UINT16	printheadSize;									2

UINT8	prCfgStatus;									1

Total:	4

PrConfig: (Inquire)

UINT8	prEvent;

UINT8	pad0;

UINT16	prConfigType;									2

UINT8	deviceSignature[SystemDevSignatureSize];

UINT8	printerType; 									1

UINT8	labelWidth;									1

UINT8	mediaSensorAdjustment;								1

UINT8	outOfMediaCount;	// Immutable!! Set to 200 on Ultima			1

UINT16	printContrast;		// "Print Contrast" on Ultima Configuration.			2

UINT16	configWord0;			// Used on "European" labels			2

UINT16	configWord1;			// Used on "European" labels			2

UINT16	configWord2;		// Used on "European" labels				2

UINT16	configWord3;		// "Expel Position" on Ultima Configuration.			2

UINT16	configWord4;			// Used on "European" labels			2

UINT16	configWord5;									2

UINT16	configWord6;									2

UINT16	configWord7;									2

UINT16	configWord8;									2

UINT16	configWord9;									2

UINT16	configWord10;									2

UINT16	configWord11;									2

UINT16	configWord12;									2

UINT16	configWord13;									2

UINT16	mediaSensorType;								2

UINT16	slaEjectLine;									2

UINT16	printPosition;									2

Total:	42

PrStatus:

UINT8	prEvent;

UINT8	pad0;

UINT8	state;										1

UINT8	command;									1

UINT8	error;										1

UINT8	history;										1

UINT8	sensor;										1

UINT8	user;										1

UINT16	counter;										1

UINT8	sensorMask;									1

UINT8	userMask;									1

UINT8	sensor2Mask;									1

UINT8	pad1;

UINT8	sla;										1

UINT8	sensor2;										1

Total:	12

PrSensors:

UINT8	prEvent;

UINT8	pad0;

UINT16	headupReading;									2

UINT16	labelWidthReading;								2

UINT16	regulatorTemperatureReading;							2

UINT16	headTemperatureReading;							2

UINT16	headVoltageReading;								2

UINT16	headCurrentReading;								2

UINT16	labelHighAverage; 								2

UINT16	labelLowAverage;								2

UINT16	labelThreshold; 									2

UINT16	labelReading; 									2

UINT16	mediaHighAverage;								2

UINT16	mediaLowAverage;								2

UINT16	mediaThreshold;									2

UINT16	mediaReading; 									2

Total:	22

System Messages:

SysWakeup:		No longer Needed!

UINT8	function_code;

UINT8	pad0;

UINT8	sysCfgStatus;									Total: 1

PrSysStatus:	Replaced by Error message and version message

UINT8 	function_code;

UINT8  	pad0;

UINT16	bit_error;										2

UINT16	address_error;										2

UINT16	rom_checksum;										2

UINT32	flags;											4

UINT8	partno[SystemPartNumberSize];								10

UINT16	system_checksum;									2

UINT32	init_flags;										4

UINT16	software_major_revision;									2

UINT16	software_minor_revision;									2

UINT8	hardware_major_revision;  // Device ID							1

UINT8	hardware_minor_revision;								1

Total:	32

PrSysConfig:		No longer Needed

UINT8	function_code;

UINT8	pad0;

UINT16 configType;										2

UINT8	device_signature[SystemDevSignatureSize];

DeviceAddress	station_address;

DeviceAddress	controller_address;

UINT8	hardware_major_revision;  // Device ID							1

UINT8	hardware_minor_revision;								1

Total: 	4

SysFlashStatus:			No longer needed

UINT8	function_code;

UINT8 	pad0;

UINT16	BootCodeMajorRevision;									2

UINT16 BootCodeMinorRevision;									2

UINT16 BootStoredChecksum;									2

UINT16 BootCalculatedChecksum;									2

UINT16 FLUD1MajorRevision;									2

UINT16 FLUD1MinorRevision;									2

UINT16 FLUD1StoredChecksum;									2

UINT16 FLUD1CalculatedChecksum;								2

UINT16 FLUD2MajorRevision;									2

UINT16 FLUD2MinorRevision;									2

UINT16 FLUD2StoredChecksum;									2

UINT16 FLUD2CalculatedChecksum;								2

UINT16 PrinterMajorRevision;									2

UINT16 PrinterMinorRevision;									2

UINT16 PrinterStoredChecksum;									2

UINT16 PrinterCalculatedChecksum;								2

UINT8 	FlashManufacturerID;									1

UINT8 	FlashDeviceID;										1

UINT8 	SelectedFLUD;										1

UINT8 	FLUD1Status;										1

UINT8 	FLUD2Status;										1

UINT8 	BPUpdateFailures;									1

UINT8 	F1F2UpdateFailures;									1

UINT8 	SysConfigStatus;										1

Total: 	40

#### Message Size (Bytes)

PrConfig / PrWriteConfig:

UINT8	prEvent;

UINT8	pad0;

UINT16	prConfigType;										2

UINT8	deviceSignature[SystemDevSignatureSize];

UINT8	printerType;										1

UINT8	labelWidth;										1

UINT8	mediaSensorAdjustment;									1

UINT8	outOfMediaCount;									1

UINT16	printContrast;										2

UINT16	configWord0;										2

UINT16	configWord1;										2

UINT16	configWord2;										2

UINT16	configWord3;										2

UINT16	configWord4;										2

UINT16	configWord5;										2

UINT16	configWord6;										2

UINT16	configWord7;										2

UINT16	configWord8;										2

UINT16	configWord9;										2

UINT16	configWord10;										2

UINT16	configWord11;										2

UINT16	configWord12;										2

UINT16	configWord13;										2

UINT16	mediaSensorType;									2

UINT16	slaEjectLine;										2

UINT16	printPosition;										2

Total:	42

PrFlush:											Total:	1

PrReset:											Total:	1

PrEnable:										Total:	1

PrDisable:										Total:	1

PrMode:											Total:	1

UINT8	prEvent;

UINT8	pad0;

UINT8	mode;										Total:	1

PrReqVersion:										Total:	1

PrVersion:

UINT16	software_major_revision;									2

UINT16	software_minor_revision;									2

UINT8	hardware_major_revision;  								1

UINT8	hardware_minor_revision;								1

Total:	6

PrError:

UINT8	error_type										1

UINT8	error_data[7]										7

Total:	8

PrPower:

UINT8	prEvent;

UINT8	pad0;

UINT16	powerState;										2

UINT16	scProtection;										2

Total:	4

PrCommand:

UINT8	prEvent;

UINT8	pad0;

UINT8	printerCommandType;									1

UINT8	printerCommandOptions;									1

UINT16	printerIndirection; 									2

UINT16	value;											2

Total:	6

PrRam:				Alex: This message is no longer needed.

UINT8	prEvent;

UINT8	pad0;

IndirectDataItem

UINT16	location;

UINT16	value;

ram[PrNumRamLocations]; 				Total:	512

PrTest:

UINT8	prEvent;

UINT8	pad0;

UINT8	data; 										Total: 	1

UINT8	pad1;

PrNomSize:

UINT8	prEvent;

UINT8	pad0;

UINT16	actualSize;										2

UINT16	measuredSize;										2

Total:	4

PrMask:

UINT8	prEvent;

UINT8	pad0;

UINT8	sensorMask;

UINT8	userMask;

UINT8	sensor2Mask;									Total:	3

UINT8	pad1;

** Move Teach tables into the Printer which would eliminate this message!!!!

PrTeach:

UINT8	prEvent;

UINT8	pad0;

UINT16	entries;											2

UINT8	command;										1

UINT8	pad1;

CommandOperationMsg	operation[30];

UINT8 commandDirective;										1

UINT8 pad[7];

UINT8 commandDirective;										1

UINT8 pad[7];

UINT8	commandDirective;									1

UINT8	type; 											1

UINT16	data; 											2

UINT8	labelFeedDirection;									1

UINT8	pad[3];

UINT8	commandDirective;									1

UINT8	type; 											1

UINT16	data; 											2

UINT8	pad[4];

UINT8	commandDirective;									1

UINT8	type; 											1

UINT16	data;											2

UINT8	bitOperation; 										1

UINT8	bits;											1

UINT8	result;											1

UINT8	pad;

UINT8	commandDirective;									1

UINT8	type; 											1

UINT16	data; 											2

UINT8	pad[4];

UINT8	commandDirective;									1

UINT8	bitOperation; 										1

UINT8	bits;											1

UINT8	result;											1

UINT8	pad[4];

UINT8	commandDirective;									1

UINT8	bitOperation; 										1

UINT8	bits;											1

UINT8	result;											1

UINT8	trueBits;											1

UINT8	falseBits;										1

UINT8	trueAction;										1

UINT8	falseAction;										1

UINT8	commandDirective;									1

UINT8	bitOperator;										1

UINT8	bits;											1

UINT8	pad[5];

UINT8	commandDirective;									1

UINT8	counterOperator;										1

UINT8	pad[6];										Total:	1260

PrReqStatus:										Total:	1

PrReqConfig:										Total:	1

PrReqWakeup:										Total:	1

PrEnableTakeup:										Total:	1

PrReqSensors:										Total:	1

PrReqCalibrate:										Total:	1

PrReqLabelTransfer:

UINT8	prEvent;

UINT8	pad0;

UINT16	bufferSize;									Total: 	2

The SAM7x CAN interface has 8 mailboxes which can be programmed for TX/RX and a range of COB ID’s can be assigned to each RX mailbox.

#### CAN Messages:

Size:

PrReqWakeup:	COB ID:	100

PrWakeup  	COB ID:	101							7

PrReqStatus:	COB ID: 	102

PrStatus		COB ID: 	103							12

COB ID: 	11F

PrReqConfig:	COB ID:	104

PrConfig 	COB ID:	105	( 6 frames)					42

COB ID: 	105

COB ID:	105

COB ID:	105

COB ID: 	105

COB ID: 	105

PrWriteConfig 	COB ID:	106							42

COB ID: 	106

COB ID:	106

COB ID:	106

COB ID: 	106

COB ID: 	106

PrReqSensors:	COB ID:	107

PrSensors	COB ID:	108							22

COB ID: 	108

COB ID:	108

PrFlush		COB ID:	109							1

PrReset		COB ID:	10A							1

PrEnable	COB ID:	10B							1

PrDisable	COB ID:	10C							1

PrMode		COB ID:	10D							1

PrPower		COB ID:	10E							4

PrCommand	COB ID:	10F							6

PrTest		COB ID:	110							1

PrNomSize	COB ID:	111							4

PrMask		COB ID:	112							3

PrEnableTakeup:	COB ID:	113							1

PrReqCalibrate: 	COB ID:	114							1

PrReqLabelTransfer:	COB ID:	115						2

PrReqVersion	COB ID: 	116							1

PrVersion	COB ID:	117							6

PrLabelTransferStatus	COB ID: 118						1

#### Can Message Format:			MSB -> LSB

## Wrapper

#### To Wrapper

Size

WrReqWakeup									Total: 	1

WrEnable									Total:	1

WrDisable									Total:	1

WrReset										Total:	1

WrRequestStatus									Total:	1

WrReqConfig(Inquire)								Total: 	1

WrWriteConfig (WrEvolConfig)

UINT16      functionCode;

UINT16      disposition;									2

UINT16      configVersion;									2

UINT16      pad;

UINT32   checksum;									4

UINT16   isUsed;										2

INT16	maxTemp;									2

INT16     minTemp;									2

UINT16  defaultTempSetting;								2

UINT16   isUsed;										2

INT16	maxTemp;									2

INT16     minTemp;									2

UINT16  defaultTempSetting;								2

UINT32  checksum;									4

UINT16  rollWidth;									2

UINT16  rollDiameter;									2

UINT16  remainingOnRoll;									2

UINT16  rollWidth;									2

UINT16  rollDiameter;									2

UINT16  remainingOnRoll;									2

INT16	wrapperOperation;								2

UINT16  filmTensionNetDefault;								2

UINT16 	addOnHardware;									2

INT16     filmTensionServiceAdjust;								2

UINT16	maxPackageWeight;				//not in use on AWS -- ats

UINT16	minPackageWeight;				//not in use on AWS -- ats

UINT16	maxPackageLength;				//not in use on AWS -- ats

UINT16	minPackageLength;				//not in use on AWS -- ats

UINT16	maxPackageWidth;				//not in use on AWS -- ats

UINT16	minPackageWidth;				//not in use on AWS -- ats

UINT16	maxPackageHeight;				//not in use on AWS -- ats

UINT16	minPackageHeight;				//not in use on AWS -- ats

UINT32  packageGrandTotal;								4

UINT32  wrapperCycleTotal;								4

UINT16  filmTensionFrontToBack;								2

UINT16  filmTensionRightToLeft;								2

UINT32  checksum;									4

UINT16	autoRotation;									2

UINT16  priLabelSizeDefault;								2

UINT16  priLabelWidth;									2

UINT16  priLabelRotationDefault;								2

UINT16  priLabelLocationDefault;								2

INT16     priLabelOffsetOnPkg;								2

UINT16	priRotationCalibrationOffset;							2

UINT16  secLabelSizeDefault;								2

UINT16  secLabelWidth;									2

UINT16  secLabelRotationDefault;								2

UINT16  secLabelLocationDefault;								2

INT16     secLabelOffsetOnPkg;								2

UINT16	secRotationCalibrationOffset;							2

UINT16	secondCLAAttached;								2

UINT16  sideDischargeType;              							2

Total:	82

WrWrap

UINT16  wrEvent;

UINT16  intakeMode;									2

UINT16  packageId; 									2

UINT16  pullFilm;  									2

UINT16  packageWeight;									2

UINT16	pad;

UINT32	pluNumber;									4

Total:	12

WrMode

UINT16      wrEvent;

UINT16      mode;										2

Total: 	2

WrControl

UINT16	wrEvent;

UINT16   sealerBeltTempSetting;								2

UINT16   sealerBeltHeaterEnabled;								2

UINT16   filmSupplyTempSetting;								2

UINT16   filmSupplyHeaterEnabled;								2

UINT16   filmTension;									2

UINT16   filmWidthSelection;								2

INT16	filmTensionFrontToBack;								2

INT16   	filmTensionRightToLeft;								2

UINT16  intakeMode;									2

Total: 	12

WrLoad										Total: 	0

UINT16  wrEvent;

UINT16	filmWidthSelection;							Total: 	2

WrRequestPackageTimings

UINT16      wrEvent;

UINT16      param;								Total:	2

**Wrap box timing messages Carlos will define**

WrSavePackageTimings		TBD

UINT16 wrEvent;

UINT16 disposition;									2

UINT16 wrapBoxes;									2

WrTest

UINT16   wrEvent;

UINT16   testNum;									2

UINT16	data1;										2

UINT16	data2;										2

UINT32	dataLong;  									4

Total:	10

WrReqSensors									Total:	1

WrRequestCycleCount								Total:	1

WrSaveCycleCount

UINT16 wrEvent;

UINT16	pad;

UINT32 cycleCount;								Total:	4

WrReqSystemInfo								Total:	1

WrSystemInfoErase								Total:	1

WrSystemInfoWrite

UINT16  wrEvent;

UINT8	awsSerialNumber[12];								12

UINT8	awsDateOfManufacture[6];							  6

UINT8	awsDateOfInstall[6];								  6

UINT16	awsModelNumber;								  2

UINT32	awsCycleCount;									  4

Total:	30

WrReqSystemReset								Total:	1

WrReqVersion

UINT16	nodeId										2

#### From Wrapper

WrWakeup

UINT16 wrEvent;

UINT16 allNodeAvailable;									2

UINT16 wrCfgStatus;									2

UINT16 wrapperOperation;								2

Total:	2

WrStatus		(message will be broken up)

UINT16 wrEvent;

UINT16 reportStatus;									2

UINT16 openWindow;									2

UINT16 limpHome;									2

UINT16 wrapperStopped;									2

UINT16 mainPower;									2

UINT16 mainMotor;									2

UINT16 wrapperState;									2

UINT16 primaryConveyor;			**current error state				2

UINT16 secondaryConveyor;								2

UINT16 secondaryShift;									2

UINT16 elevator;										2

UINT16 filmSupply;									2

UINT16 gripper;										2

UINT16 knife;										2

UINT16 sideClamp;									2

UINT16 sideFolder;									2

UINT16 rearFolder;									2

UINT16 frontFolder;									2

UINT16 pusher;										2

UINT16 sealerBelt;									2

UINT16 labelApplier;									2

UINT16 generalWrapper;									2

UINT16 primaryConveyor;									2

UINT16 secondaryConveyor;								2

UINT16 secondaryShift;									2

UINT16 elevator;										2

UINT16 filmSupply;									2

UINT16 gripper;										2

UINT16 knife;										2

UINT16 sideClamp;									2

UINT16 sideFolder;									2

UINT16 rearFolder;									2

UINT16 frontFolder;									2

UINT16 pusher;										2

UINT16 sealerBelt;									2

UINT16 labelApplier;									2

UINT16 generalWrapper;									2

UINT16 wrapperMode;									2

UINT16 sealerBeltTemperature;								2

UINT16 sealerBeltSetting;									2

UINT16 sealerBeltStatusBits;								2

UINT16 sealerBeltHeaterEnabled;								2

UINT16 filmTemperature; 									2

UINT16 filmSetting;									2

UINT16 filmStatusBits;									2

UINT16 filmTempEnabled;									2

UINT16 ambientTemperature;								2

UINT16 ambientTemperatureStatusBits;							2

EvolWrNodeInternalTemperature nodeBoardTemperature[MaxNodeBoards];

UINT16 internalTemperatureFailed;

Total:	28

WrError

UINT16 Mechanism									2

UINT16 Error Description									2

WrOpen										Total:	1

WrConfig

UINT16   functionCode;

UINT16	disposition;									2

UINT16   configVersion;									2

UINT16   pad;

UINT32	checksum									4

UINT16   isUsed;										2

INT16      maxTemp;									2

INT16	minTemp;									2

UINT16  defaultTempSetting;								2

UINT16  isUsed;										2

INT16     maxTemp;									2

INT16     minTemp;									2

UINT16	defaultTempSetting;								2

UINT32	checksum;									4

UINT16  rollWidth;									2

UINT16  rollDiameter;									2

UINT16  remainingOnRoll;									2

UINT16  rollWidth;									2

UINT16  rollDiameter;									2

UINT16  remainingOnRoll;									2

INT16	wrapperOperation;								2

UINT16  filmTensionNetDefault;								2

UINT16 	addOnHardware;									2

INT16     filmTensionServiceAdjust;								2

UINT16	maxPackageWeight;				//not in use on AWS -- ats

UINT16	minPackageWeight;				//not in use on AWS -- ats

UINT16	maxPackageLength;				//not in use on AWS -- ats

UINT16	minPackageLength;				//not in use on AWS -- ats

UINT16	maxPackageWidth;				//not in use on AWS -- ats

UINT16	minPackageWidth;				//not in use on AWS -- ats

UINT16	maxPackageHeight;				//not in use on AWS -- ats

UINT16	minPackageHeight;				//not in use on AWS -- ats

UINT32  packageGrandTotal;								4

UINT32  wrapperCycleTotal;								4

UINT16  filmTensionFrontToBack;								2

UINT16  filmTensionRightToLeft;								2

UINT32	checksum;									4

UINT16	autoRotation;									2

UINT16  priLabelSizeDefault;								2

UINT16  priLabelWidth;									2

UINT16  priLabelRotationDefault;								2

UINT16  priLabelLocationDefault;								2

INT16	priLabelOffsetOnPkg;								2

UINT16	priRotationCalibrationOffset;							2

UINT16  secLabelSizeDefault;								2

UINT16  secLabelWidth;									2

UINT16  secLabelRotationDefault;								2

UINT16  secLabelLocationDefault;								2

INT16    	secLabelOffsetOnPkg;								2

UINT16	secRotationCalibrationOffset;							2

UINT16	secondCLAAttached;								2

UINT16  sideDischargeType;      								2

Total:	82

WrSysStatus

UINT16	wrEvent;

UINT16   pad0;

UINT16   report_sys_error;								2

UINT16   pad1;

UINT16   nodeid; (**** Times 6 **** 1 for each node)					2

UINT16   config_version;                        							2

UINT16   device_init_flags;								2

UINT16 	major;										2

UINT16 	minor;  										2

UINT16 	major;										2

UINT16 	minor; 										2

UINT16 	major;										2

UINT16 	minor;  										2

UINT16   pad0;

UINT32   error_flags;									4

UINT32   wrap_box_checksum;    								4

Total 26 *6+2 	158

WrVersion

UINT16 nodeid										2

UINT16 config_version 									2

UINT16 wrap_box_checksum								2

UINT16 boot_major									2

UINT16 boot_minor									2

UINT16 data_major									2

UINT16 data_minor									2

UINT16 app_major									2

UINT16 app_minor									2

Total: 	16

WrPkgSettingsRead									TBD

WrPkgSettingsWriteComplete								TBD

WrTestComplete

UINT16	wrEvent;

UINT16	testNum;									2

UINT16	data1;										2

UINT16	data2;										2

UINT32	dataLong;									4

UINT16	passed;										2

Total:	12

WrSense

UINT16	wrEvent;

UINT16	pad;

UINT32	switchesSensors;									4

UINT16	interlockSwitches;								6

Total:	10

WrPkgDesc

UINT16	wrEvent;

UINT16  id;										2

UINT16  length; 										2

UINT16	width;										2

UINT16  height;										2

UINT16	lip;										2

UINT16	filmWidthUsedToWrap;								2

UINT16	wrapBoxIndex;									2

UINT16  heightBoxIndex;									2

UINT16  usedWrapBoxBestFit;								2

UINT16  modifiedNetStretch;								2

Total:	20

WrEventCapture

UINT16 wrEvent;

UINT16 eventCaptureType;								2

UINT16 packageId;									2

UINT16 pad;

UINT32 eventTime;									4

UINT16 pad1;

UINT16 timeLimitExceeded;								2

Total:	10

WrErrHistory

UINT16 wrEvent;

UINT16 wrError;										2

UINT8 trayName[20];									20

UINT16 packageLength;									2

UINT16 packageWidth;									2

UINT16 packageHeight;									2

UINT16 filmRollUsed;									2

UINT16 usedWrapBoxBestFit;								2

UINT16 modifiedNetStretch;								2

Total:	34

WrWrapCycleCount

UINT16 wrEvent;

UINT16	pad;

UINT32 cycleCount;								Total:	4

WrTransferStationDischarging

UINT16      wrEvent;

UINT16      param;								Total: 	2

WrSysInfo

UINT16  wrEvent;

UINT16	validBoardChecksum;								2

UINT8	motorBoardPartNumber[10];							10

UINT8	motorBoardRev[2];								2

UINT8	motorBoardDateOfManufacture[6];						6

UINT16	validAwsChecksum;								2

UINT8	awsSerialNumber[12];								12

UINT8	awsDateOfManufacture[6];							6

UINT8	awsDateOfInstall[6];								6

UINT16	awsModelNumber;								2

UINT16	pad0;

UINT32	awsCycleCount;									4

Total:	52

WrManufacturingRuninNotice							Total:	1

WrMotorProfile

UINT16 wrEvent;

UINT16 bldcMechanism;									2

UINT16	motorStatus;									2

UINT16 motorCapacityUsage;								2

Total:	6

WrLoaderStatus

UINT16      wrEvent;

UINT16      nodeId;									2

UINT16      status;										2

UINT16      fault;										2

UINT16      updateFailureCount;     								2

UINT16      softComponent;  								2

UINT32      addr;										4

Total:	14

Reformat of WrConfig

NGW_Config

UINT8 Disposition									1

UINT16 configVersion									2

UINT32 checksum									4

INT16 rollwidth										2

INT16 wrapperOperation									2

UINT16 filmTensionNetDefault								2

UINT16 autoRotation									2

UINT16 addOnHardware									2

UINT16 filmTensionFrontToBack								2

UINT16 filmTesionRightToLeft								2

UINT16 secondCLAAttached								2

UINT16 sideDischargeType								2

Total: 	25

New Messages

NGW_ApplierConfig

UINT8 applierPosition	(primary, secondary)						1

UINT16 labelWidth									2

UINT16 defaultLabelRotation								2

UINT16 defaultLableLocation								2

INT16 labelOffset										2

UINT16 rotationCalibrationOffset								2

Total:	11

NGW_SealerBeltConfig

UINT8 isUsed										1

INT16 maxTemp										2

INT16 minTemp										2

Total:	5

NGW_CycleTotal

UINT32 packageGranTotal									2

UINT32 wrapperCycleTotal								2

Total:	4

WrLabelReady									Total:	4

UINT16	Station ID;

UINT16  Package ID;

WrLabelTaken									Total:	4

UINT16	StationID;

UINT16  Package ID;

WrLabelSetup

UINT16	laEvent;										2

UINT16	Station ID;									2

UINT16  LabelSize									2

UINT16	LabelRotation;									2

UINT16	LabelLocation;									2

Total:	8

WrWrapperUpdate

UINT16      wrEvent;									2

UINT16      node1;									2

UINT16      update1;									2

UINT16      node3;									2

UINT16      update3;									2

UINT16      node4;									2

UINT16      update4;									2

UINT16      node5;									2

UINT16      update5;									2

UINT16      node6;									2

UINT16      update6;									2

UINT16      node2;									2

UINT16      update2;									2

Total:	24

## Wrapper

Cob ID 500 – 700

WrWakeup				COBID:500

WrStatus				COBID:596

COBID:597

COBID:598

COBID:599

WrConfig 				COBID:588

COBID:589

COBID:58A

COBID:58B

COBID:58C

WrWriteConfig (WrEvolConfig)		COBID:568

COBID:569

COBID:56A

COBID:56B

COBID:56C

WrCycle					COBID:506

WrOpen					COBID:59d

WrWrap					COBID:530

COBID:531

WrMode				COBID:539

WrVersion				COBID:50A

COBID:50A

WrControl				COBID:53A

COBID:53B

WrLoad					COBID:511

WrTest					COBID:579

COBID:57a

WRSensors				COBID:5a7

WrPkgDesc				COBID:534

COBID:535

COBID:536

WrEventCapture				COBID:594

WrWrapCycleCount			COBID:518

WrReqCycleCount			COBID:522

WrSaveCycleCount			COBID:517

WrReqSystemInfo			COBID:566

WrSysInfo				COBID:5B0

COBID:5B1

COBID:5B2

COBID:5B3

COBID:5B4

COBID:5B5

COBID:5B6

WrSystemInfoErase			COBID:51B

WrSystemInfoWrite			COBID:51D

WrReqSystemReset			COBID:51F

WrManufactureRuninNotice		COBID:5B7

WrMotorProfile				COBID:570

WrRequestPackageTimings		COBID:521

WrPkgSettingsRead			COBID:522

WrPkgSettingsWrite			COBID:523

WrPkgSettingsWriteComplete		COBID:524

WrErrHistory				COBID:5A9

COBID:5AA

COBID:5AB

WrLabelReady				COBID:532

WrLabelTaken				COBID:533

WrLabelSetup				COBID:53C

WrLoaderStatus				COBID:5EF

COBID:5EF

WrWrapperUpdate			COBID:5F1

COBID:5F1

COBID:5F1

WrRequestWrapBoxAll		COBID:541

WrRequestWrapBox		COBID:554

WrWrapBox			COBID:540

WrWrapBoxComplete		COBID:542

WrRequestHeightBox		COBID:545

WrHeightBox			COBID:544

WrHeightBoxComplete		COBID:546

WrChangeParam		COBID:547

WrChangeComplete		COBID:548

WrTimingsSave			COBID:549

WrTimingsSaveComplete	COBID:54A

#### Can Message Format:			MSB -> LSB

| WrPkgDesc: |  |  |  |  |
| --- | --- | --- | --- | --- |
| 0x534 | id | length | width | height |
|  | B0 - B1 | B2 -B3 | B4 - B5 | B6 - B7 |
| 0x535 | lip | filmWidthToWrap | wrapboxIndex | heightboxIndex |
|  | B0 - B1 | B2 -B3 | B4 - B5 | B6 - B7 |
| 0x536 | usedBestFit | modNetStretch |  |  |
|  | B0 - B1 | B2 -B3 |  |  |

## Wrap Boxes:

- 28

## Host Wrap Box Read:

## Host Wrap Box Write:

## Scale Backend Design:

## RS485:

This interface is used to transfer the label data image to the printer  due to lack of bandwidth on the CAN Bus.

The interface is configured in Asynchronous mode at a baud rate of 3Mb/s.  The following is label transfer times using the RS485 interface at 3Mb/s.

Preliminary Results:

Label     |              Time (mS)

3”x4”     |              ~225

3”x5”     |              ~290

3”x6”     |              ~335

3”x8”     |              ~450

Transfer of 3 eight inch labels the different printers with in a wrap cycle would require 1.350 Seconds to complete. This time is within our 2 Second spec.

## Unit Test Data Scale Backend:

#### Can Manager Probe:

[Main] Startup CanManager:

[CanManager] Entered CanManager() constructor

[CanManager] Socket ready!!!

[CanTranslator] CanTranslator()

[CanTranslator] Register for Events

[CanLibrary] CanLibrary(): build device libraries.

[CanLibrary] buildWeigherLibrary()

[CanLibrary] buildPrinter1Library()

[CanLibrary] buildPrinter2Library()

[CanLibrary] buildPrinter3Library()

[CanLibrary] buildWrapperLibrary()

[CanManager] initializeTranslator(): initialized!

[CanManager] probeBus()

[CanTranslator] sendWeigherRequest()

[CanLibrary] getWeigherCobID(): 34

[CanTranslator] sendWeigherRequest(): Sending CAN frame: 0400

[CanManager] send():

[CanTranslator] sendPrinterRequest()

[CanTranslator] sendPrinterRequest(): Send CAN Frame: 0100

[CanManager] send():

[CanTranslator] sendPrinterRequest()

[CanTranslator] sendPrinterRequest(): Send CAN Frame: 0200

[CanManager] send():

[CanTranslator] sendPrinterRequest()

[CanTranslator] sendPrinterRequest(): Send CAN Frame: 0300

[CanManager] send():

[CanTranslator] sendWrapperRequest()

[CanTranslator] translateWrapperRequest() Send CAN Frame: 0501

[CanManager] send():

CAN Sniffer:

root@beaglebone-carlos:~# candump can0

can0  400   [1]  00 	|

can0  100   [1]  00	|

can0  200   [1]  00	| <-- Bus Probe

can0  300   [1]  00	|

can0  501   [1]  00	|

#### Weigher Down Stream Messages:

#### Printer Down Stream Messages:

#### Wrapper Down Stream Messages:

#### Printer Up Stream Messages:

#### Weigher Up Stream Messages:

#### Wrapper Up Stream Messages:

[CanTranslator.WrEvolStatus] ReportStatus: 0			can0  502   [8]  00 00 00 01 00 00 00 00

[CanTranslator.WrEvolStatus] OpenWindow: 1			can0  502   [8]  00 01 00 00 00 00 00 00

[CanTranslator.WrEvolStatus] LimpHome: 0				can0  502   [8]  01 5E 01 5E 00 00 00 01

[CanTranslator.WrEvolStatus] WrapperStopped: 0			can0  502   [4]  00 46 00 00

[CanTranslator.WrEvolStatus] MainPower: 1

[CanTranslator.WrEvolStatus] WrapperState: 0

[CanTranslator.WrEvolStatus] WrapperMode: 0

[CanTranslator.WrEvolStatus] SealerBeltTemp: 0

[CanTranslator.WrEvolStatus] SealerBeltSetting: 350

[CanTranslator.WrEvolStatus] SealerBeltStatusBits: 350

[CanTranslator.WrEvolStatus] SealerBeltHeaterEnabled: 0

[CanTranslator.WrEvolStatus] AmbientTemp: 1

[CanTranslator.WrEvolStatus] AmbientTempStatusBits: 70

[CanTranslator.WrEvolInquire] Checksum: 305419896		can0  504   [8]  00 00 01 01 12 34 56 78

[CanTranslator.WrEvolInquire] IsUsed: 1				can0  504   [8]  00 01 00 96 00 7D 00 96

[CanTranslator.WrEvolInquire] MaxTemp: 150			can0  504   [8]  00 01 01 5E 01 45 01 5E

[CanTranslator.WrEvolInquire] MinTemp: 125			can0  504   [8]  00 00 04 D2 00 00 00 00

[CanTranslator.WrEvolInquire] DefaultTempSetting: 150		can0  504   [8]  00 00 00 01 00 05 00 14

[CanTranslator.WrEvolInquire] IsUsed: 1				can0  504   [8]  00 00 2D 05 00 03 0D 40

[CanTranslator.WrEvolInquire] MaxTemp: 350			can0  504   [8]  00 00 00 00 00 EC 75 70

[CanTranslator.WrEvolInquire] MinTemp: 325			can0  504   [8]  00 02 04 56 52 50 00 00

[CanTranslator.WrEvolInquire] DefaultTempSetting: 350		can0  504   [8]  00 00 00 00 00 00 00 00

[CanTranslator.WrEvolInquire] Checksum: 1234			can0  504   [8]  00 01 00 00 00 00 00 00

[CanTranslator.WrEvolInquire] TopRollWidth: 0			can0  504   [2]  00 00

[CanTranslator.WrEvolInquire] BottomRollWidth: 0

[CanTranslator.WrEvolInquire] WrapperOperation: 0

[CanTranslator.WrEvolInquire] HardwareAddOn: 1

[CanTranslator.WrEvolInquire] FilmTensionFrontToBack: 5

[CanTranslator.WrEvolInquire] FilmTensionRightToLeft: 20

[CanTranslator.WrEvolInquire] PackageGrandTotal: 11525

[CanTranslator.WrEvolInquire] WrapperCycleTotal: 200000

[CanTranslator.WrEvolInquire] Checksum: 0

[CanTranslator.WrEvolInquire] HasAutoRotation: 1

[CanTranslator.WrEvolInquire] PriLabelSizeDefault: 30064

[CanTranslator.WrEvolInquire] PriLabelWidth: 2

[CanTranslator.WrEvolInquire] PriLabelRotationDefault: 1110

[CanTranslator.WrEvolInquire] PriLabelLocationDefault: 21072

[CanTranslator.WrEvolInquire] PriLabelOffsetOnPkg: 0

[CanTranslator.WrEvolInquire] PriRotationCalibrationOffset: 0

[CanTranslator.WrEvolInquire] SecLabelSizeDefault: 0

[CanTranslator.WrEvolInquire] SecLabelWidth: 0

[CanTranslator.WrEvolInquire] SecLabelRotationDefault: 0

[CanTranslator.WrEvolInquire] SecLabelLocationDefault: 1

[CanTranslator.WrEvolInquire] SecLabelOffsetOnPkg: 0

[CanTranslator.WrEvolInquire] SecRotationCalibrationOffset: 0

[CanTranslator.WrEvolInquire] IsSecondCLAAttached: 0

[CanTranslator.WrEvolInquire] SideDischargeType: 0

[CanTranslator.WrEvolPackageDesc] Id: 100

[CanTranslator.WrEvolPackageDesc] Length: 650

[CanTranslator.WrEvolPackageDesc] Width: 400			can0  514   [8]  00 64 02 8A 01 90 00 C8

[CanTranslator.WrEvolPackageDesc] Height: 200			can0  514   [8]  00 64 00 96 00 01 00 02

[CanTranslator.WrEvolPackageDesc] Lip: 100			can0  514   [4]  00 01 00 00

[CanTranslator.WrEvolPackageDesc] WrapBoxIndex: 1

[CanTranslator.WrEvolPackageDesc] HeightBoxIndex: 2

[CanTranslator.WrEvolPackageDesc] UsedWrapBoxBestFit: 1

[CanTranslator.WrEvolPackageDesc] ModifiedNetStretch: 0

[CanTranslator.WrEvolKnifeTime] EventCaptureType: 0 		can0  516   [8]  00 00 00 64 00 00 02 6C

[CanTranslator.WrEvolKnifeTime] PackageId: 100			can0  516   [2]  00 96

[CanTranslator.WrEvolKnifeTime] EventTime: 620

[CanTranslator.WrEvolKnifeTime] TimeLimitExceeded: 1

[CanTranslator.WrEvolTestComplete] TestCode: 7			can0  510   [8]  00 07 00 00 00 00 00 01

[CanTranslator.WrEvolTestComplete] Data1: 0			can0  510   [4]  00 00 00 96

[CanTranslator.WrEvolTestComplete] Data2: 0

[CanTranslator.WrEvolTestComplete] TestPassed: 1

[CanTranslator.WrNgwCycleCount] Cycle Count: 150000		can0  518   [4]  00 02 49 F0

[CanTranslator.WrEvolErrorHistory] TrayName: 5S			can0  52E   [8]  00 0F 35 53 00 00 00 00

[CanTranslator.WrEvolErrorHistory] PackageLength: 400		can0  52E   [8]  00 00 00 00 00 00 00 00

[CanTranslator.WrEvolErrorHistory] PackageWidth: 700		can0  52E   [8]  00 00 00 00 00 00 01 90

[CanTranslator.WrEvolErrorHistory] PackageHeight: 550		can0  52E   [8]  02 BC 02 26 00 01 00 00

[CanTranslator.WrEvolErrorHistory] FilmUsed: 1			can0  52E   [2]  00 00

[CanTranslator.WrEvolErrorHistory] UsedBestFit: 0

[CanTranslator.WrEvolErrorHistory] StretchModified: 0

[CanTranslator.WrNgwVersion] NodeId: 1				can0  50A   [8]  00 01 01 01 12 34 00 05

[CanTranslator.WrNgwVersion] WrapBoxChecksum: 4660		can0  50A   [8]  00 03 00 04 00 00 00 05

[CanTranslator.WrNgwVersion] ConfigVersion: 01.01			can0  50A   [2]  00 07

[CanTranslator.WrNgwVersion] BootVersion: 05.03

[CanTranslator.WrNgwVersion] DataVersion:04.00

[CanTranslator.WrNgwVersion] AppVersion:05.07

[CanTranslator.WrNgwError] Mechanism: 3				can0  520   [4]  00 03 00 10

[CanTranslator.WrNgwError] Error: 16

[CanTranslator.WrOpen] Wrapper Open Window			can0  508   [1]  00

[CanTranslator.WrManufacturingRunIn] WrManufacturingRunIn	can0  530   [1]  00

[CanTranslator.WrEvolSystemInfo] Part# offset: 2			can0  51A   [8]  00 01 34 34 35 34 34 30

[CanTranslator.WrEvolSystemInfo] Rev offset: 9			can0  51A   [8]  30 30 36 30 31 31 36 31

[CanTranslator.WrEvolSystemInfo] BoardChecksumStatus: 1		can0  51A   [8]  32 00 01 30 31 32 33 2D

[CanTranslator.WrEvolSystemInfo] MotorBoardPartNumber: 4454400	can0  51A   [8]  34 35 36 37 30 39 31 31

[CanTranslator.WrEvolSystemInfo] MotorBoardRevision: 06		can0  51A   [8]  31 34 30 39 31 31 31 35

[CanTranslator.WrEvolSystemInfo] MotorBoardDateOfManufacture: 011612 	can0  51A   [6]  00 07 00 00 00 64

[CanTranslator.WrEvolSystemInfo] NGWChecksumStatus: 1

[CanTranslator.WrEvolSystemInfo] NGWSerialNumber: 0123-4567

[CanTranslator.WrEvolSystemInfo] NGWDateOfManufacture: 091114

[CanTranslator.WrEvolSystemInfo] NGWDateOfInstall: 091115

[CanTranslator.WrEvolSystemInfo] ModelNumber: 7

[CanTranslator.WrEvolSystemInfo] NGWCycleCount: 100

## Added extra byte to message Id: 0x532 to check error handling for translator dequque.

[CanTranslator.Error] Message Overrun Id: 0532 Message Size: 10 Total Bytes Received: 11

[CanTranslator] Message Complete, Erase from Queue: 0532

#### Wrapper Firmware Upgrade:

#### UNIT Test Data:	 No update files loaded on scale

can0  540   [1]  01			<-- [NGWUpdateManager] Send request for status to node: 1

can0  54B   [7]  01 00 00 00 00 00 00	<-- [NGWUpdateManager] Node loader initialized. Switching state to: _RequestVersion

can0  54C   [8]  22 FF 01 00 00 00 00 00	<-- [NGWUpdateManager] Send request for version info to node: 1

can0  54C   [8]  00 01 02 04 00 00 00 00	<-- [NGWUpdateManager] Version message received for node: 1 Application version: 2.256

can0  543   [8]  00 00 00 00 00 00 00 02

can0  544   [8]  01 00 03 00 01 00 01 00

can0  545   [8]  01 00 00 00 00 00 00 00

can0  546   [8]  00 00 00 00 00 00 00 00	<-- [NGWUpdateManager] No source image found.

can0  541   [3]  01 7C 03		<-- [NGWUpdateManager] Starting node: 1

can0  540   [1]  03			<-- [NGWUpdateManager] Send request for status to node: 3

can0  54B   [7]  03 00 00 00 00 00 00	<-- [NGWUpdateManager] Node loader initialized. Switching state to: _RequestVersion

can0  54C   [8]  22 FF 03 00 00 00 00 00	<-- [NGWUpdateManager] Send request for version info to node: 3

can0  54C   [8]  00 03 02 04 00 00 00 00	<-- [NGWUpdateManager] Version message received for node: 3 Application version: 2.256

can0  543   [8]  00 00 00 00 00 00 00 02

can0  544   [8]  01 00 03 00 01 00 01 00

can0  545   [8]  01 00 00 00 00 00 00 00

can0  546   [8]  00 00 00 00 00 00 00 00	<-- [NGWUpdateManager] No source image found.

can0  541   [3]  03 7C 03		<-- [NGWUpdateManager] Starting node: 3

can0  540   [1]  04			<-- [NGWUpdateManager] Send request for status to node: 4

can0  54B   [7]  04 00 00 00 00 00 00	<-- [NGWUpdateManager] Node loader initialized. Switching state to: _RequestVersion

can0  54C   [8]  22 FF 04 00 00 00 00 00	<-- [NGWUpdateManager] Send request for version info to node: 4

can0  54C   [8]  00 04 02 04 00 00 00 00	<-- [NGWUpdateManager] Version message received for node: 4 Application version: 2.256

can0  543   [8]  00 00 00 00 00 00 00 02

can0  544   [8]  01 00 03 00 01 00 01 00

can0  545   [8]  01 00 00 00 00 00 00 00

can0  546   [8]  00 00 00 00 00 00 00 00	<-- [NGWUpdateManager] No source image found.

can0  541   [3]  04 7C 03		<-- [NGWUpdateManager] Starting node: 4

can0  540   [1]  05			<-- [NGWUpdateManager] Send request for status to node: 5

can0  54B   [7]  05 00 00 00 00 00 00	<-- [NGWUpdateManager] Node loader initialized. Switching state to: _RequestVersion

can0  54C   [8]  22 FF 05 00 00 00 00 00	<-- [NGWUpdateManager] Send request for version info to node: 5

can0  54C   [8]  00 05 02 04 00 00 00 00	<-- [NGWUpdateManager] Version message received for node: 5 Application version: 2.256

can0  543   [8]  00 00 00 00 00 00 00 02

can0  544   [8]  01 00 03 00 01 00 01 00

can0  545   [8]  01 00 00 00 00 00 00 00

can0  546   [8]  00 00 00 00 00 00 00 00	<-- [NGWUpdateManager] No source image found.

can0  541   [3]  05 7C 03		<-- [NGWUpdateManager] Starting node: 5

can0  540   [1]  06			<-- [NGWUpdateManager] Send request for status to node: 6

can0  54B   [7]  06 00 00 00 00 00 00	<-- [NGWUpdateManager] Node loader initialized. Switching state to: _RequestVersion

can0  54C   [8]  22 FF 06 00 00 00 00 00	<-- [NGWUpdateManager] Send request for version info to node: 6

can0  54C   [8]  00 06 02 04 00 00 00 00	<-- [NGWUpdateManager] Version message received for node: 6 Application version: 2.256

can0  543   [8]  00 00 00 00 00 00 00 02

can0  544   [8]  01 00 03 00 01 00 01 00

can0  545   [8]  01 00 00 00 00 00 00 00

can0  546   [8]  00 00 00 00 00 00 00 00	<-- [NGWUpdateManager] No source image found.

can0  541   [3]  06 7C 03		<-- [NGWUpdateManager] Starting node: 6

can0  540   [1]  02			<-- [NGWUpdateManager] Send request for status to node: 2

can0  54B   [7]  02 00 00 00 00 00 00	<-- [NGWUpdateManager] Node loader initialized. Switching state to: _RequestVersion

can0  54C   [8]  22 FF 02 00 00 00 00 00	<-- [NGWUpdateManager] Send request for version info to node: 2

can0  54C   [8]  00 02 02 04 00 00 00 00	<-- [NGWUpdateManager] Version message received for node: 2 Application version: 2.256

can0  543   [8]  00 00 00 00 00 00 00 02

can0  544   [8]  01 00 03 00 01 00 01 00

can0  545   [8]  01 00 00 00 00 00 00 00

can0  546   [8]  00 00 00 00 00 00 00 00	<-- [NGWUpdateManager] No source image found.

can0  541   [3]  02 7C 03		<-- [NGWUpdateManager] Starting node: 2											      [NGWUpdateManager] Finished.											      [NGWUpdateManager] Canceled timer.											      [NGWUpdateManager] Exiting thread

#### Upgrade file for node 1 loaded on scale (version 2.256) version matching

can0  540   [1]  01			<-- [NGWUpdateManager] Send request for status to node: 1

can0  54B   [7]  01 00 00 00 00 00 00	<-- [NGWUpdateManager] Node loader initialized. Switching state to: _RequestVersion

can0  54C   [8]  22 FF 01 00 00 00 00 00	<-- [NGWUpdateManager] Send request for version info to node: 1

can0  54C   [8]  00 01 02 04 00 00 00 00	<-- [NGWUpdateManager] Version message received for node: 1 Application version: 2.256

can0  543   [8]  00 00 00 00 00 00 00 02

can0  544   [8]  01 00 03 00 01 00 01 00

can0  545   [8]  01 00 00 00 00 00 00 00

can0  546   [8]  00 00 00 00 00 00 00 00	<-- [NGWUpdateManager] Versions match.

can0  541   [3]  01 7C 03		<-- [NGWUpdateManager] Starting node: 1

#### Upgrade file for node 1 loaded on scale (version 2.257) only for node upgrade

#### Wrapper bootloader

(011.999675)  can0  540   [1]  04				<-- Send request for status to node: 4

(011.999746)  can0  54B   [7]  04 00 00 00 00 00 00		--> Node 4 status

(012.001914)  can0  54C   [8]  22 FF 04 00 00 00 00 00	<-- Send request for version info node: 4

(012.001988)  can0  54C   [8]  23 04 02 00 00 00 00 00		--> Node 4 version info

(012.002138)  can0  54C   [8]  00 00 00 00 00 00 00 01		--> Node 4 version info

(012.002163)  can0  54C   [8]  00 00 FF FF FF FF 00 03		--> Node 4 version info

(012.002330)  can0  54C   [8]  00 00 00 00 00 00 00 00		--> Node 4 version info

(012.002470)  can0  54C   [8]  00 00 00 00 00 00 00 00		--> Node 4 version info

(012.002837)  can0  541   [3]  04 7C 02			<-- Send command to upgrade flash

(012.012616)  can0  54B   [7]  04 01 00 00 00 00 00		--> Node 4 status ready. ( flash erased without errors )

(012.014608)  can0  542   [8]  1F FF 04 08 00 00 00 00	<-- Send bulk transfer header

(012.014964)  can0  543   [8]  54 E1 B6 1B 54 E1 B6 1B	<-- bytes 0 - 7

(012.015247)  can0  544   [8]  54 E2 F5 08 54 E2 F5 08	<-- bytes 8 - 15

(012.015564)  can0  545   [8]  54 E2 F5 08 54 E2 9D 09	<-- bytes 16 - 23

(012.015819)  can0  546   [8]  54 E2 F5 08 54 E2 F5 08	<-- bytes 24 - 31

(012.016087)  can0  547   [8]  54 E2 F5 08 54 E2 F5 08	<-- bytes 32 - 39

(012.016366)  can0  548   [8]  54 E2 F5 08 54 E2 F5 08	<-- bytes 40 - 47

(012.016805)  can0  549   [8]  54 E2 F5 08 54 E2 F5 08	<-- bytes 48 - 55

(012.017114)  can0  54A   [8]  54 E2 F5 08 54 E2 F5 08	<-- bytes 56 - 63

(012.017568)  can0  54D   [4]  04 FF 00 00			--> Node 4 Ack (programmed bulk transfer to flash without error)

(012.018179)  can0  54E   [2]  04 FF			<-- Send Flush command (get ready for next transfer)

16K bytes of data transferred

(012.848759)  can0  542   [8]  1F FF 04 08 00 00 00 00	<-- Send bulk transfer header

(012.849038)  can0  543   [8]  40 86 05 02 18 E4 2E F0

(012.849339)  can0  544   [8]  00 00 40 86 A1 0D 18 E4

(012.849608)  can0  545   [8]  36 F0 00 00 40 86 04 00

(012.849879)  can0  546   [8]  18 E4 23 F3 00 00 40 86

(012.850150)  can0  547   [8]  10 02 18 E4 38 F0 00 00

(012.850452)  can0  548   [8]  40 86 5C 82 18 E4 03 F3

(012.850738)  can0  549   [8]  00 00 40 86 07 00 54 E1

(012.851010)  can0  54A   [8]  D0 00 08 E7 00 00 00 00

(012.851463)  can0  54D   [4]  04 FF 00 00

Upgrade time ~= .848 Seconds for 16k bytes of data.

Node flash size: 512k bytes

Total node upgrade time: ( 512 / 16 ) .848  = 27.136 seconds per node

Nodes per machine: 6

Total machine upgrade time:  ( 6 * 27.136s ) / 60s  = 2.713 Minutes

#### Uint Test

Invalid Node ID: 		Remove node id jumper on the node board.

can0  54B   [7]  0F 02 00 00 00 00 0A	-- > status message ( state = FAULT,  fault = Node id error )

Code / Node ID mismatch:	Set node jumper to some valid id other than it’s original id. So if code for node 4 is loaded set the jumper id to 3.

can0  54B   [7]  03 02 00 00 00 00 08 	-- > status message ( state = FAULT,  fault = Node image mismatch)

Invalid Image checksum:	Modify the application binary so image does not match calculated checksum.

can0  54B   [7]  04 02 00 00 00 00 09	-- > status message ( state = FAULT,  fault = Image checksum mismatch)

Bootloader timeout:		Tell the bootloader to update but do not send image to allow the loader to time out.

can0  54B   [7]  04 02 00 00 00 00 01	-- > status message ( state = FAULT,  fault = Timeout)

Bootloader reset:		Tell the bootloader to reset.

can0  541   [3]  01 7C 04

Jump to application: 		Instruct bootloader to start application.

Program and jump to:		Send valid image to bootloader and instruct to start.

Application