This procedure describes how to install Ubuntu 9.10 to a CompactFlash card on the HLX console.  A custom installation DVD is used, which results in a smaller installed disk footprint.

Materials Required:

- HLX Console

- USB 2.0 DVD reader.

- 4GB CompactFlash (CF) card

- USB keyboard and mouse

- DVD with Ubuntu Custom Installation ISO.

  - The .iso file can be obtained here:

ftp://ftp.bsquare.com/ubuntu-9.10-desktop-i386-custom1.0.iso

Username: Hobart

Password: [request from project lead]

  - For information on how to burn the .ISO file to DVD, see the following. https://help.ubuntu.com/community/BurningIsoHowto

Procedure:

- Power off the target and disconnect the target AC power cable.

- Remove the hard disk assembly from the target.

- Install a 4GB CompactFlash (CF) card to the target. CF label faces toward rear.

- Attach a USB keyboard and mouse to the target.

- Attach the USB DVD reader to the target. Power on the reader and insert the installation DVD.

- Power on the target.

- Press the “Delete” key until the System BIOS Setup screen appears.

- Select “Reset to factory defaults” and confirm. Target restarts.

- Press the “Delete” key until the System BIOS Setup screen appears.

- Manually configure BIOS settings

  - Select “Basic CMOS Configuration” and verify the following

    - DRIVE ASSIGNMENT ORDER – Drive C: Ide 0/Pri Master, all others (None)

    - BOOT ORDER – Boot 1st: CDROM, Boot 2nd: Drive C:, all others (None)

    - Press Escape to return to the System BIOS Setup screen.

  - Select “Features Configuration” and verify the following

    - ACPI 1.0:  Enabled

    - Advanced Power Management: Enabled

    - Primary IDE UDMA: Enabled

    - Secondary IDE UDMA: Disabled

    - AtaMassStorage: Disabled

    - UsbMassStorage: Enabled

    - Usb20: Enabled

    - Press Escape to return to the System BIOS Setup screen.

  - Select “Custom Configuration” and verify the following

    - Legacy USB Support: Enabled

    - USB Device Controller:Disabled

    - USB Port 4 Function: Host

    - Press Escape to return to the System BIOS Setup screen.

  - Select “Save CMOS settings” and confirm. Target restarts.

- The target now boots from the installation DVD.

- The installer Language selection screen appears with the cursor pre-positioned on “English”. Press Enter to accept. The installer top-level menu is displayed.

- Press down arrow once to highlight “Install Ubuntu”. Do NOT press Enter at this time.

- Press F6 “Other Options”.

  - Highlight the “acpi=off” selection and Press Enter.  Verify an “x” appears next to the selection.

  - Highlight the “noapic” selection and Press Enter.  Verify an “x” appears next to the selection.

  - Press Escape to clear the “Other Options” menu.

- Press Enter to launch the installer. A pulsing logo displays while the installer is loading and initializing. The Ubuntu graphical installation is fully documented here: https://help.ubuntu.com/community/GraphicalInstall.  The remainder of this procedure will only document specific variations and options which are required.

- At the “Prepare Disk Space” screen

  - Verify the selected disk is “SCSI1 (0,0,0) (sda) – 4.0 GB ATA CF Card” or similar.

  - Choose the option labeled “Erase and use the entire disk”.

- At the “Who are you?” screen

  - What is your name?  “hobart”

  - What name do you want to use to log in? “hobart” (lower case)

  - Choose a password … “hobart”

  - What is the name of this computer? “hlx-NNNN” where NNNN is any arbitrary value. Note this only needs to be unique if you will be connecting multiple HLX consoles to the same network subnet. If unsure, coordinate with the team lead.

  - Choose “Log in automatically”.

  - When the “Use Strong Password?” dialog appears,  click Continue.

- At the “Ready to Install” screen

  - Click “Advanced”. The Advanced dialog appears.

  - Confirm “Install boot loader” is checked.

  - Under “Device for boot loader installation”, choose “/dev/sda” from the drop list.

  - Click OK.  The Advanced dialog clears.

  - Click Install

- Installation proceeds.

- When the Installation Complete dialog appears,  click “Restart Now”.   The display switches to text mode.

- The DVD reader will eject the installation DVD and you should see the prompt “Please remove the disc and close the tray (if any) then press ENTER.”  Remove the installation DVD from the reader and press Enter to restart.

- When the HLX splash screen appears, press and hold the shift key until the GRUB bootloader displays a boot menu.

- Highlight the top selection and press ‘e’ to edit. The GRUB command list editor is displayed. This text is editable using the cursor keys, delete, backspace, etc.

- Identify the line that begins with “linux” and append “acpi=noirq” at the end of the line.

- Press Ctrl-X to boot the system.

- After the system boots, open a terminal window (Applications|Accessories|Terminal).

- Edit the grub default boot options as follows.

  - sudo gedit /etc/default/grub   (sudo password is ‘hobart’)

  - identify the line beginning with  GRUB_CMDLINE_LINUX_DEFAULT

  - add the text “ acpi=noirq” within the quoted string. Note the space before the “acpi”.

  - Save the file.

  - sudo update-grub

- Restart the system. (menu power icon| Restart…)

[end]