This document captures optimizations for Ubuntu 9.10 booted from CompactFlash card on the HLX console.  The purpose of these optimizations is twofold: minimize writes to the CompactFlash card, and increase runtime performance.

Materials Required:

- HLX Console

- Ubuntu 9.10 installed to 4GB CompactFlash (CF) card

- USB keyboard and mouse

Optimizations:

- Modify fstab (gksudo gedit /etc/fstab)

  - Redirect /tmp and /var to ram. Add the following lines at the bottom of the file.

tmpfs /tmp         tmpfs defaults,noatime,mode=1777        0 0

none  /var/tmp     aufs noatime,br:/tmp=rw:/var/tmp=ro     0 0

none  /var/run     aufs noatime,br:/tmp=rw:/var/run=ro     0 0

none  /var/log     aufs noatime,br:/tmp=rw:/var/log=ro     0 0

none  /var/lock    aufs noatime,br:/tmp=rw:/var/lock=ro    0 0

none  /var/cache   aufs noatime,br:/tmp=rw:/var/cache=ro   0 0

  - Optimize ext4 settings for the primary partition.

    - barrier: The default value is barrier=1 (activated). Deactivating write barriers improves the performance. NOTE: If you don't have a battery-backed storage environment, you should not deactivate this mount option.

    - noatime:  Deactivates the file access time modification.

    - commit: Increases the commit/write interval (default: 5s).

    - nouser_xattr:  Disable Extended User Attributes.

    - data=writeback: The default data mode is ordered. This mode provides only meta-data journaling. This option can only be enabled on non-root partitions.

    - nobh: Implies data=writeback. Avoids buffer heads to data pages association.

UUID=”blahblahblah” /

ext4 noatime,barrier=0,nobh,commit=100,nouser_xattr 0 1

- Modify /etc/rc.local

  - Disable write-back caching

hdparm –W0 /dev/sda

  - Set I/O scheduler mode ( mode := noop | deadline ?)

echo mode > /sys/block/sda/queue/scheduler

echo 1 > /sys/block/sda/queue/iosched/fifo_batch

- Modify /etc/sysctl.conf

vm.swappiness = 0

vm.dirty_ratio = 40

vm.dirty_background_ratio = 1

vm.dirty_writeback_centisecs = 24000

vm.dirty_expire_centisecs = 0

- (optional) Configure Firefox browser to cache its data in ram.

  - open Firefox and type “about:config” in the browser window

  - click “I’ll be careful, I promise!”

  - create key “browser.cache.disk.parent_directory” and set to “/tmp”.

  - Use “about:cache” to verify the setting.

References:

https://help.ubuntu.com/community/Fstab

http://lxr.linux.no/#linux+v2.6.34/Documentation/filesystems/tmpfs.txt

http://lxr.linux.no/#linux+v2.6.34/Documentation/filesystems/ext4.txt

http://www.thinkwiki.org/wiki/Compact_Flash_boot_drive

http://forums.linuxfordevices.com/embedded-linux-questions-answers-f3/linux-file-system-on-compact-flash-t907.html

https://wiki.ubuntu.com/KernelTeam/Specs/KarmicSSD

http://www.zdnet.com/blog/perlow/geek-sheet-a-tweakers-guide-to-solid-state-drives-ssds-and-linux/9190

http://wiki.geteasypeasy.com/How_to:_Reduce_Disk_Writes_to_Prolong_the_Life_of_your_Flash_Drive

http://stevehanov.ca/blog/index.php?id=48

http://en.dogeno.us/2010/01/karmic-with-solid-state-disk-how-to-optimize-ubuntu-for-ssd/

https://help.ubuntu.com/community/EeePC/Using

http://aspireonezg5.wordpress.com/2009/10/16/karmic-howto/

http://blog.loxal.net/2009/04/tuning-ext4-for-performance-with.html

http://tombuntu.com/index.php/2007/11/15/benchmark-your-linux-system-with-hardinfo/

http://thunk.org/tytso/blog/2009/03/01/ssds-journaling-and-noatimerelatime/

http://www.brighthub.com/computing/linux/articles/9170.aspx